3GPP TSG-SA3 Meeting #114e ad-hoc	S3-240303

Electronic meeting, online, 22 - 26 January 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0179 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GSMA review - Test Case on No Directory Listings |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, BSI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The GSMA NESAS has reviewed and proposed changes to the existing test case via the document S3-234423. The proposed changes in this document are with reference to Slide#38. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GSMA suggested changes: Define what is or is not to be used as an automatic assessment tool. Define what needs to be checked and what are the expected settings. As for other similar tests, these are repeating the requirement, rather than specifying the steps of the testing that is required for the test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test results and misinterpretation of test execution steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial draft and presentation |  |  |  |  |  |  |  |  |

******************** Start of 1st Change ************************************************************

#### 4.3.4.10	No directory listings

Requirement Name: No directory listings / Directory Browsing.

Requirement Reference: In accordance with industry best practice

Requirement Description: Directory listings (indexing) / "Directory browsing" shall be deactivated.

Threat References: TR 33.926 [4]

Test Case:

Test Name: TC_NO_DIRECTORY_LISTINGS

Purpose:

To verify that Directory listings / Directory browsing has been deactivated in all Web server components.

Procedure and execution steps

Pre-Conditions:

-	The tester has administrative privileges

-	A tester machine is available.

-	Recommended: an automatic assessment tool has been configured / script adapted in line with the Requirement Description.

Execution Steps

-	The tester checks the web server configuration for Directory listings (indexing) / "Directory browsing" to be deactivated in all web server components.

Note1: Whether directory listings have been deactivated could be done by checking the webserver configuration file specifically the parameters related to directory listing. The directory listing should be turned off in the web server configuration file, and there is no activation capability.

Note2: Directory listings could be obtained by entering a valid URL (e.g., /var/www/test_1) that does not contain any index file.

Expected Results:

-	Directory listing / Directory browsing has been deactivated in all Web server components.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

-	Log files and screen shots of test executions

-	Text excerpt of the web server configuration showing that directory listing is disabled

-	Test result (Passed or not)

******************** End of 1st Change ************************************************************