3GPP TSG-SA3 Meeting #115 	S3-240322

# Athens, Greece, 26 February - 1 March 2024

# Source:	KDDI Corporation, THALES

# Title:	New Key Issue on insufficient long-term key length for 256-bit security

# Document for:	Approval

Agenda Item:	5.5 New Study on enabling a cryptographic algorithm transition to 256-bits

# 1	Decision/action requested

Approve the pCR to TR 33.700-41

# 2	References

[1]			3GPP TS 31.102

# 3	Rationale

This contribution proposes a new key on insufficient long-term key length for 256-bit security.

# 4	Detailed proposal

For SA3 to accept this proposal.

*** Start of 1st Change ***

## 5.X	Key Issue #X:

### 5.X.1	Key issue details

According to TS 31.102 [1], it is still possible to provision current USIMs with long-term keys of 128-bit length. Regardless of their length, the long-term secret stored in the USIM and the UDM is used to derive CK an IK (and following child keys) of 256-bit length.

Assuming both sides support them, the UE and the network may agree on the use of 256-bit cryptographic algorithms for AS and/or NAS, even though the long-term secret is only 128-bit long. This creates an issue, because the long-term secret does not provide sufficient entropy. Thus, despite 256-bit keys being used on AS and/or NAS layer, the subscriber may not benefit from "true" 256-bit security.

### 5.X.2	Threats

256-bit cryptographic keys derived from a 128-bit secret do not provide "true" 256-bit security due to a lack of sufficient entropy, which is the threat of insufficient cryptographic strength (Threat 3)

There is a threat of a false sense of security when 256-bit cryptographic algorithms are used with keys derived from a 128-bit long term secret.

There is a threat of Denial of Service due to usage of more computationally intensive cryptography or lower performance throughput by exhaustion of resources at the gNB and AMF (Threat 2).

### 5.X.3	Potential security requirements

Based on operator policy, 256-bit cryptographic algorithms should be used only when the long-term keys are of 256-bit length or higher.

5G System shall provide a mechanism to allow or disallow the use of 256-bit cryptographic algorithms with a 128-bit long term secret.

*** End of 1st Change ***