3GPP TSG-SA3 Meeting SA3#115	S3-240476

Athens, Greece, 26th February – 1st March 2024

# Source:	vivo, China Mobile

# Title:	Discussion Paper on Study on security aspects of Core Network Enhanced Support for AIML

# Document for:	Endorsement

Agenda Item:	6

1	Decision/action requested

This contribution proposes to discuss security aspects of Core Network Enhanced Support for AIML.

2	References

[1]	SP-231753	“New SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)”

[2]	TR 23.700-84	“Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)”

[3]	3GPP TR 38.843 “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

[4]	RP-234039	“New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[5]	SP-231784	“New SID on enablers for Zero Trust Security”

[6]	S3-235110	“LS on Model Sharing With MTLF”

[7]	3GPP TR 33.866 “Study on security aspects of enablers for Network Automation (eNA) for the 5G system (5GS) Phase 2”

3	Rationale

3.1	Introduction

The SA2 Rel-19 AI/ML study is endorsed in SP-231753 [1], the study is to investigate and identify potential core network enhancement to support Artificial Intelligence (AI)/Machine Learning (ML). The latest TR 23.700-84 [2] has 4 related key issues (WT1.4, WT2, WT3.1, WT3.2) and ? use cases (WT1.4, WT2, WT3.1, WT3.2).

The objective includes the following:

“

-	WT#1: AI/ML cross-domain coordination aspects

Study enhancements to support AI enabled RAN based on conclusions of the RAN study in 3GPP TR 38.843. The WT will discuss whether and how to support the cross domain (i.e. UE, RAN, 5GC, OAM and AF) collaborative AI/ML mechanisms for the aspects described by the work tasks below.

-	WT1.1 –Study whether and how to support UE data collection to meet requirements for RAN AI support for air interface operation (for RAN) for UE-side model training documented in 3GPP TR 38.843 clause 7.2.1.3.2. This includes identifying what benefit can be achieved from enhanced UE data collection in 5G System, and the potential impacts on the 5G framework, including potential enhancements to policy control and OAM. The WT will also discuss the possible data leakage from the operator’s domain which should be avoided and the network control over data collection.

-	WT1.2 –Study whether (and how) to support model transfer/delivery to the UE according to RAN1/RAN2 considerations documented in 3GPP TR 38.843 clause 7.1.2.4, including potential enhancements to policy control and OAM. Whether and what entities or functions transfer the AI/ML model or information to the UE will be studied as part of the work. This WT will also discuss the possible data leakage from the operator’s domain which should be avoided.

-	WT1.3: Study whether and how to support the alignment of model identification and model management between SA2 and RAN. Work will be based on the possible requirements defined by RAN1 and RAN2 considering the conclusions in 3GPP TR 38.843.

-	WT1.4: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning considering the conclusions in 3GPP TR 38.843.

NOTE 1: The work will not modify the architectural principle that a service-based architecture only applies for 5GC.

NOTE 2: Whether SA2 can start work on WT 1.1, 1.2 and 1.3 will be discussed at SA#105 (Sep. 2024) based on the outcome of the related work in the involved RAN WGs(s). Further change in description of WT 1.1, 1.2, 1.3 can be discussed at SA#105.

NOTE 3: UE data collection, model delivery and transfer to the UE and model identification/management are not within the scope of WT#1.4.

NOTE 4: Further alignment with SA5 for the AI/ML Functional framework and charging may be required.

NOTE 5: security aspects are in the scope of SA3, however architectural aspects related to security enhancements will be discussed in this WT.

NOTE 6: The model management will follow the framework as defined by RAN.

- WT2:  Study whether and what potential enhancements are needed to enable 5G system to assist in collaborative AI/ML operation involving 5GC/NWDAF and/or AF for “Vertical Federated Learning (VFL)”. The work will be based only on and limited to the scope of justified use cases.

NOTE 7: RAN and UE aspects are out of scope. Solutions based on interactions between the application client and 5GS are out of scope. The necessary communication between AF and UE application client to support the collaborative AI/ML operation is understood as no normative procedure impact. Horizontal FL procedure defined in R18 should be taken into account and reused whenever possible.

NOTE 8: coordination with SA6 is required.

- WT3: Study enhancements to support NWDAF-assisted policy control and address network abnormal behaviour

- WT3.1 – Study whether and what additionally needs to be supported in order to enhance 5GC NF operations (i.e. policy control and QoS) assisted by NWDAF. The work will firstly identify the specific use cases to be considered, in order to identify the appropriate scope. The work will analyse the result impacts on NWDAF (e.g. the need to understand specific NF functionality), and the compatibility of new solutions wrt existing analytics, in order to determine the need and benefits of new solutions.

- WT3.2 – Study prediction, detection, prevention, and mitigation of network abnormal behaviours i.e. signalling storm with the assistance of NWDAF.

NOTE 9: The study will focus primarily on existing enforcement mechanisms when available and identify new ones when no existing ones can be used.

NOTE 10: The study will consider the study/work done by SA WG5 and CT WG4 in this regard already and collaborate with SA WG5/CT WG4 regarding the handling of abnormal network behaviours.

”

3.2	Security analysis on SA2 Work Task

3.2.1	WT#1 AI/ML cross-domain coordination aspects

For WT1.1 - study whether and how to support UE data collection to meet requirements for RAN AI support for air interface operation (for RAN) for UE-side model training documented in 3GPP TR 38.843 [3] clause 7.2.1.3.2, as stated in the SP-231753 [1], the privacy issue for UE data collection should be considered.

For WT1.2 - Study whether (and how) to support model transfer/delivery to the UE according to RAN1/RAN2 considerations documented in 3GPP TR 38.843 [3] clause 7.1.2.4, and WT1.3 - Study whether and how to support the alignment of model identification and model management between SA2 and RAN, since the model may be transferred among UE, RAN and CN, the authorization, confidentiality and integrity of ML model should also be considered.

Observation 1: Regarding WT 1.1 – 1.3, privacy issue for UE data collection and authorization, confidentiality and integrity of ML model should be considered. However, according NOTE2, the SA2 may decide to start the related work at SA#105 (Sep. 2024), SA3 should also wait for RAN and SA2 progress.

For WT1.4 - Study whether and how to consider enhancements to LCS to support AI/ML based Positioning considering the conclusions in 3GPP TR 38.843 [3]. RAN approved WID RP-234039 [4] regarding AI/ML for NR Air Interface, the positioning related objectives are approved as below:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

There are 5 use cases (i.e. case 1, 2a, 2b, 3a, 3b) which will be studied by RAN, and as agreed in TR 23.700-84 [2], only case 2b and case 3b (i.e. model is on the LMF) will be studied at this stage, and the main issue is to study model transition between LMF and NWDAF. Thus, the authorization of ML model retrieval should be considered.

For case 1 and 2a, the model is located in UE side, and for case 3a, the model is located in gNB side. There is no ML model transition, the only issue may be privacy of collected data, so it is based on RAN conclusion.

Observation 2: Regarding WT 1.4, authorization of ML model should be considered. SA3 could start to study authorization of ML model retrieval by LMF for case 2b and 3b. For other cases (i.e. case 1, 2a, 3a), SA3 should wait for RAN and SA2 progress.

3.2.2	WT#2 Vertical Federated Learning (VFL)

Federated learning is a distributed machine learning technique that trains machine learning models collaboratively among multiple independent training entities, each using its own dataset. There are two general categories of federated learning: horizontal federated learning (which is supported among network data forwarding (NWDAF) nodes in Rel-18) and vertical federated learning (which is being studied for Rel-19).

Vertical Federated Leaning (VFL) is an essential technique for cross-domain AI/ML framework. For example, VFL will address the following challenges.

-Blocker of data usage: Mobile networks consisting of multiple domains, such as CN and AF, generate vast and beneficial data. Instead, some of them have yet to be fully utilized.

- The first blockage is private implementations; each domain has private implementation but is unwilling to report its data.

- Second is the privacy-sensitive data, such as UE location. These raw data cannot be exposed due to the operator's or government's privacy protection policy.

- Needs diversity: Defining analytics for each application is impractical due to the diversity of application needs. Now, AF has to put up with the uniform standardized Analytics output.

One of the VFL architecture is shown as below in figure 1.

[source: https://arxiv.org/pdf/2107.03428.pdf]

Figure 1 Example of Architecture for a vertical federated learning system

- Suppose that companies A and B would like to jointly train a machine learning model, and their business systems each have their own data. In addition, Company B also has label data that the model needs to predict.

- For data privacy and security reasons, A and B cannot directly exchange data. In order to ensure the confidentiality of the data during the training process, a third-party collaborator C is involved.

- We assume the collaborator C is honest and does not collude with A or B, but party A and B are honest-but-curious to each other.

See details via Federated Machine Learning: Concept and Applications https://arxiv.org/pdf/1902.04885.pdf

Another architecture for vertical federate learning is shown below, where no collaborator is needed as in Figure 2.

Figure 2 Example of Architecture for a (split) vertical federated learning system (training and inference)

- Suppose that companies A, B and C, that are represented by the different locations in Figure 2, would like to jointly train a machine learning model, and their business systems each have their own data.

- For data privacy and security reasons, A, B and C cannot directly exchange data. In order to ensure the confidentiality of the data, inter-layer information (weight, gradients) is exchanged between different parts of a single model.

- Training cycles are repeated until the model converges.

It should be noted that for the first architecture (i.e. with collaborator), key distribution and encryption will be considered.

Besides, similar like HFL, FL group will also be considered in SA2, thus, authorization issue should also be considered.

Observation 3: Regarding WT 2, key distribution and encryption for collaborator case should be consider, besides, authorization of FL group should be considered too.

3.2.3	WT#3 NWDAF-assisted policy control and address network abnormal behaviour

It seems that network abnormal behaviour detection is covered by work task of eZTS study as depicted in SP-231784 [5], this study will not cover it in order to avoid redundant work.

Observation 4: Regarding WT 3, since network abnormal behaviors detection is covered by work task of eZTS study, this study will not cover it.

3.3	Security analysis on leftover from R18

We have discussed authorization issue of ML model retrieval between MTLF and ADRF and does not reach a conclusion in SA3#113 meeting due to limited time, finally we sent LS S3-235110 [6] to SA2 to tell them SA3 has acknowledged the security issue and SA3 doesn’t agree a security solution. If we cannot agree the security solution in R18 due to lack of time, then we may study this procedure in R19 to address the security issue.

Observation 5: A new WT is needed in R19 to study authorization issue of ML model retrieval between MTLF and ADRF if SA3 cannot agree a solution in R18.

3.4	Security analysis on AI security

AI will be the essential feature for 6G, native AI is a very hot topic for 6G. However, SA3 does not carefully discuss AI security itself. In R17, SA3 has one related key issue#1.2 and one related solution #8 in 3GPP TR 33.866 [7], it is concluded that AI security itself is out of 3GPP scope.

However, SA3 does not carefully analysis existing AI security specification which is out of 3GPP, e.g. specifications from ETSI SAI, ISO/IEC JTC1 SC42, etc., to see whether security requirements in those specification should be adopted as security requirement for NWDAF or have architecture impact.

Thus, it is proposed to analysis the existing AI security specifications, and to study whether some of the security requirements of those AI security specifications should be referred as security requirements to AI related to network functions, e.g. NWDAF, and be good prepared for 6G AI security.

Observation 6: A new WT is needed in R19 to analysis the existing AI security specifications, and to study whether some of the security requirements of those AI security specifications should be referred as security requirements to AI related to network functions, e.g. NWDAF.

4	Detailed proposals

Based on the observations above:

Observation 1: Regarding WT 1.1 – 1.3, privacy issue for UE data collection and authorization, confidentiality and integrity of ML model should be considered. However, according NOTE2, the SA2 may decide to start the related work at SA#105 (Sep. 2024), SA3 should also wait for RAN and SA2 progress.

Observation 2: Regarding WT 1.4, authorization of ML model should be considered. SA3 could start to study authorization of ML model retrieval by LMF for case 2b and 3b. For other cases (i.e. case 1, 2a, 3a), SA3 should wait for RAN and SA2 progress.

Observation 3: Regarding WT 2, key distribution and encryption for collaborator case should be consider, besides, authorization of FL group should be considered too.

Observation 4: Regarding WT 3, since network abnormal behaviors detection is covered by work task of eZTS study, this study will not cover it.

Observation 5: A new WT is needed in R19 to study authorization issue of ML model retrieval between MTLF and ADRF if SA3 cannot agree a solution in R18.

Observation 6: A new WT is needed in R19 to analysis the existing AI security specifications, and to study whether some of the security requirements of those AI security specifications should be referred as security requirements to AI related to network functions, e.g., NWDAF.

We propose:

Proposal:  a new SID is needed to study security aspects of core network enhanced support for AI/ML, and the objectives should cover observation 1 - 6.