3GPP TSG-SA3 Meeting #115	S3-240522

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | DraftCR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test for local certificate checking at gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei; HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TBD |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of a new test for local certificate checking that corresponds to a new threat introduced in a parallel contribution S3-240526 to the gNB network product class annex in TR 33.926. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new testcase for local certificate checking by gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete SCAS coverage of the certificate enrolment functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* 1st of Change*************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	Void

[5]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[7]	3GPP TS 23.501: "System Architecture for 5G System (5GS)".

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description".

[9]	3GPP TS 33.523: "5G Security Assurance Specification (SCAS); split gNB product classes".

[10]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

*********** End of 1st of Change***********

************* 2nd of Change*************

4.2.2.1.x	Local configured certificate checking

Requirement Name: local configured certificate checking at base station

Requirement Reference: TS 33.310 [x], clause 9.2

Requirement Description:

The gNB shall have the capability to check the validity of locally stored certificates prior to usage.

Test case:

Test Name: TC_LOCAL_CERT_CHCK

Purpose:

Verify that the base station can check that locally stored or configured certificates are valid or not

Procedure and execution steps:

Pre-Conditions:

-	A UPF or AMF or a security gateway may be real/emulated serving as a peer for the gNB network product.

-	The gNB is configured the necessary information to be able to establish connection with the peer.

Execution Steps

1. The tester configures the gNB with an invalid certificate.

2. The tester tries to trigger the gNB to connect to the peer, this step is optional.

Expected Results:

-	The raise an alarm that the local certificate is invalid. The result may happen before step2.

Expected format of evidence:

The logs and the communication flow in a .pcap file.

************* End of Change*************