3GPP TSG-SA3 Meeting #115	S3-240619

# Athens, Greece, 26 February– 02 March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Security for Selective SCG Activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | .Proposes no new technical content but proposes to merge different pCRs proposed. The draft focuses on proactive solutions that ensure synchronization of keys and counters before UP message transmission begins, such as enhanced signaling mechanisms or confirmation of key and counter reception by the SN prior to UP communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed security mechanism and procedures for subsequent CPAC (SCPAC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security protection for the subsequent CPAC (SCPAC) procedure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.10.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RG	5G Residential Gateway

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G NSWO	5G Non-Seamless WLAN Offload

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUN3	Authenticable Non-3GPP devices

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

CCA	Client Credentials Assertion

Cell-ID	Cell Identity as used in TS 38.331 [22]

CH	Credentials Holder

CHO	Conditional Handover

CIoT	Cellular Internet of Things

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

cNRF	consumer's NRF

CP	Control Plane

cPLMN	consumer's PLMN

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DCS	Default Credentials Server

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EDT	Early Data Transmission

EMSK	Extended Master Session Key

EN-DC	E-UTRA-NR Dual Connectivity

ENSI	External Network Slice Information

EPS	Evolved Packet System

FN-RG	Fixed Network RG

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPUPS	Inter-PLMN UP Security

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LI	Lawful Intercept

MBSF	Multicast/Broadcast Service Function

MBSSF	Multicast/Broadcast Service Security Function

MBSTF	Multicast/Broadcast Service Transport Function

MeNB	Master eNB

MN	Master Node

MO-EDT	Mobile Originated Early Data Transmission

MT-EDT	Mobile Terminated Early Data Transmission

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G-Capable

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

NSSAA	Network Slice Specific Authentication and Authorization

NSWO	Non-Seamless WLAN Offload

NSWOF	Non-Seamless WLAN Offload Function

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

pNRF	producer's NRF

pPLMN	producer's PLMN

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

PUR	Preconfigured Uplink Resource

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SCP	Service Communication Proxy

NOTE: Void.	Security Gateway

SEPP	Security Edge Protection Proxy

SgNB	Secondary gNB

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

TSC	Time Sensitive Communication

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

# * * * * Next change * * * *

### 6.10.2.X	Security mechanism and procedures for SCPAC

### 6.10.2.X.1		General

In subsequent CPAC (SCPAC), the MN may provide one or several candidate SCG configuration(s) for one or multiple candidate SN(s) to the UE. The UE may select and execute precisely one of these conditional reconfigurations to change PSCell based on the measurement results on candidate target PSCells. The conditional reconfiguration for the selected PScell remains valid after the UE selects the target and executes the target cell access procedure. Thus, the UE can connect to the same SN several times without any further reconfiguration by the network.

Figure 6.10.2.X.1-1: Security mechanism and procedures for SCPAC

### 6.10.2.X.2		Security context initialization for selective SCPAC

To prevent key-stream reuse when the UE switches back and forth to the same PSCell or the SN, the MN assigns the sequence of multiple distinct SN Counter values (maintained for dual connectivity) per candidate SN during the subsequent CPAC (SCPAC) procedure. Each SN Counter value is unique and with non-overlapping sequences provided to the UE by the MN. The UE shall store these values.

The MN will transmit the KSN keys associated with an SN, alongside their corresponding SN Counter value sequence, to that SN within the SN Addition Request. Upon receipt, the SN stores the KSN keys and the SN Counter value sequence within its security context.

When a new AS root key, KNG-RAN, in the associated 5G AS security context is established, and the SN Counter is set to ‘0’, the MN assigns a new sequence of multiple distinct SN Counter values per candidate SN to the UE. This sequence is conveyed to the UE within the same RRC Reconfiguration that activates the new KNG-RAN. The UE shall delete the stored SN Counter values and store the received new SN Counter values. Further, MN derives the corresponding KSN for each target SN, and the derived KSN keys and the corresponding SN counter values are sent to the SN from the MN.

### 6.10.2.X.3		Security mechanism for UE to access target PSCell or SN

When the UE accesses an SCG of a target PSCell or SN for the first time or a subsequent time based on the same conditional reconfiguration, the UE shall select the first unused SN counter value from the sequence associated with the target PSCell or SN. This selection guarantees non-precedent use with the current KgNB.. The UE then derives the corresponding KSN using the SN Counter as described in Annex A.16 of this document and initiates the access procedure. The SN chooses the first unused KSN key of the UE to establish the security association with the UE.

In parallel, UE informs the MN of the SN counter value utilized for KSN derivation in RRC Connection Reconfiguration Complete. The MN, in turn, relays the corresponding SN counter value to the SN in the SN Reconfiguration Complete message.

In case of a key mismatch, the SN, having stored the KSN keys and the corresponding SN counter values, selects the appropriate KSN based on the received SN counter values for subsequent data access under the same reconfiguration. Also, should UP messages, safeguarded by encryption and integrity keys derived from the KSN, reach the SN prior to it receiving the SN Counter value, the SN may initially be unable to avert a mismatch with the UE regarding the SN Counter value and KSN. Nevertheless, the SN can leverage the SN Counter value to rectify any failure, ensuring subsequent data access under the same reconfiguration aligns correctly.

# * * * * Next change * * * *

# A.16	Derivation of KSN

This input string is used when the MN and UE derive KSN. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

Editor’s note: This Annex A.16 is for Derivation of Ksn for DC, SA3 to analyse and decide whether a new dedicated FC value to be defined for selective SCG.

# * * * * End of changes * * * *