3GPP TSG-SA3 Meeting #115	S3-240736

# Athens, Greece, 26 Feb - 1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | 0092 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a new clause in annexure to Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes specific to SMSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Indian Institute of Technology Bombay |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_SMSF |  |  |  |  |  | Date: |  |  | 19-02-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new clause in annexure with aspects specific to network product class SMSF |  |  |  |  |  |  |  |  |
| Summary of change: |  | A clause on threat related to SMSF data protection over SGd Diameter interface is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legacy interfaces assests if not protected, could lead to security breach and network attacks |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********Agreed Content in S3-240121 Online Meeting TSG-SA3 Meeting #114e************

Annex x: 
Aspects specific to the network product class SMSF

# x.y	Network product class description for the SMSF

## x.y.1	Introduction

The present document captures the network product class descriptions, threats and critical assets that have been identified in the course of the work on 3GPP security assurance specifications. The main body of the present document contains generic aspects that apply to more than one network product class, while this annexure covers the aspects specific to the SMSF network product class.

## x.y.2	Minimum set of functions defining the SMSF network product class

As part of the SMSF network product, it is expected that the SMSF contains SMSF application, a set of running processes (typically more than one) executing the software package for the SMSF functions and OAM functions that are specific to the SMSF network product model. Functionalities specific to the SMSF network product introduce additional critical assets and/or threats as described below.

Note: For the purposes of the present document, this common set is defined to be the list of functions contained in clause 6.2.13 TS 23.501 [31].

# x.z	Assets and threats specific to the SMSF

## x.z.1	Critical assets

In addition to the critical assets of a GNP described in clause 5.2 of the TS 33.926 [7] document, the critical assets specific to the SMSF to be protected are:

-	SMSF Application;

-	NF and User Data: e.g. NF capabilities and events, network and user sensitive information (e.g., UeSMSContextData like supi, gpsi, ueLocation etc.), data retrieved from UDM, etc.

-	The interfaces of SMSF to be protected and which are within SECAM scope:

-	Service based interface for providing services to AMF, SMS-GMSC, IP-SM-GW, SMS-Router

-	Service based interface for consuming services from AMF, UDM

-    Reference point interface SGd (Diameter-based) with IP-SM-GW, SMS-GMSC, SMS- router

-    MAP-based SS7 interface with IP-SM-GW/GMSC/SMS- router

-	Console interface, for local access: local interface on SMSF

-	OAM interface, for remote access: interface between SMSF and OAM system

-	SMSF Software: binary code or executable code

NOTE 2: 	SMSF files may be any file owned by a user (root user as well as non-root uses), including User account data and credentials, Log data, configuration data, OS files, SMSF application, NF and User data, or SMSF Software.

*************** End  ****************

*************** Start of the Change ****************

## x.z.2	Threats related to SMSF data

### x.z.2.1	SMSF control data and user data protection

- Threat name:  Diameter protection

- Threat Category: Tampering, Information Disclosure.

- Threat Description: SMS related traffic is transported between the SMSF and the IP-SM-GW/SMS-GMSC/SMS-Router on SGd Diameter interface. In roaming scenario, transportation is via Diameter Edge Agent (DEA) node on SGd interface. If the SMS data transported over the interface is not confidentiality protected, it can be subject to eavesdropping, and information leakage to unauthorized parties. If the SMS traffic is not integrity protected, the SMS traffic can be tampered with.

If the protection of the SGd interface is done using wrong security profiles, i.e., weak security algorithms or protocol versions with known vulnerabilities, then overall security of the data transported over SGd is compromised.

- Threatened Asset: Network function and data.

*************** End of the Change ****************