3GPP TSG-SA3 Meeting #115	S3-240745

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCPAC: Releasing prepared SCPAC configurations at handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
| G | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | KSNs in SCPAC configurations are dervied from the currently active KgNB. At handover when a new KgNB is established, those KSNs must be updated to be in sync with the current key hierarchy and obtain the same forward and backward security properties. Therefore, this pCR proposes that the handling of prepared SCPAC configurations are handeled in the same way as prepared secondary cells are handled at handover. That is, the source MN releases all prepared configurations before the handover. The target MN (re)establishes the configurations it wishes based on the new KgNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause describing that prepared SCPAC configurations are deleted when a handover updating the KgNB occur. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Keys established with SCPAC cells have worse security than the currently active KgNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.10.2.X.3	Handling prepared configurations at handover

At handover when a fresh KNG-RAN, is derived (necessary for inter-gNB handovers), the source MN shall release any prepared SCPAC configurations in candidate SNs.  Wheather a fresh KNG-RAN needs to be derived by the UE or not is part of existing signaling for handover procedure. Note that for inter-gNB HO, the source gNB (MN) must derive a new KNG-RAN. If the target MN wishes to establish new prepared SCPAC configurations, it will do so based on the fresh KNG-RAN. As specified in clause 6.10.3.1, the SN Counter is also reset to ‘0’ at such handovers.

# * * * * End of changes * * * *