3GPP TSG-SA3 Meeting #115 	S3-240784

# Athens, Greece, 26 February - 1 March 2024

# Source:	Xiaomi

# Title:	New KI on 256-bit CK/IK in AKA procedure

# Document for:	Approval

Agenda Item:	5.5

1	Decision/action requested

Approve the pCR to TR 33.700-41

2	References

[1]	3GPP TS 33.501 Security architecture and procedures for 5G system.

[2]	3GPP TS 33.220 Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)

[3]	S3‑211408 Choice of cryptographic algorithm in 256-bit Milenage

[4]	S3‑234134 Specification of Milenage-256 finalized

[5]	3GPP TS 35.231 A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm specification

[6]	IETF RFC 2104 Keyed-Hashing for Message Authentication

3	Rationale

This contribution proposes a new key issue on 256-bit CK and 256-bit IK in AKA procedure.

The concatenation of 128-bit CK and 128-bit IK is used by ME /UDM/ARPF in the following scenarios.

- To derive KAUSF in 5G AKA procedure, the concatenation of 128-bit CK and 128-bit IK (i.e. CK||IK) is used as the input key of KDF (i.e. the HMAC-SHA-256) specified in TS 33.501 [1] and TS 33.220 [2].

- To derive CK' and IK' in EAP-AKA' procedure, the concatenation of 128-bit CK and 128-bit IK (i.e. CK||IK) is used as the input key of KDF (i.e. the HMAC-SHA-256) specified in TS 33.501 [1] and TS 33.220 [2].

To mitigate potential quantum threats, MILENAGE 256-bit algorithm set or Tuak algorithm set may be used by 3GPP operators. According to S3‑211408 [3] and S3‑234134 [4], for candidate MILENAGE 256-bit algorithm set, i.e. MILENAGE-256-R, provided by SAGE, CK/IK can be 256-bit, The Tuak algorithm supporting 256-bit K may also generate 256-bit CK and 256-bit IK as specified in TS 35.231 [5].

If MILENAGE-256-R or Tuak algorithm set is selected, the USIM/UDM/ARPF will generate 256-bit CK and 256-bit IK. In the current 5GS, the traditional ME /UDM/ARPF derives the KAUSF/CK'/IK' using 128-bit CK and 128-bit IK. Moreover, the input key of HMAC-SHA-256 can be any key with length longer or equal to the length of the HMAC-SHA-256 output (i.e. 256 bits) [z]. The following statement is captured from the IETF RFC 2104 [6], in which the L is the length of the HMAC-SHA-256 output.

Keys longer than L bytes are acceptable but the extra length would not significantly increase the function strength. (A longer key may be advisable if the randomness of the key is considered weak.)

The mechanism to enable the ME/UDM/ARPF to support 256-bit CK/IK is not clear.

4	Detailed proposal

***************Start of the Change ****************

5.X	Key Issue #X: 256-bit CK/IK in AKA procedure

5.X.1	Key issue details

The concatenation of 128-bit CK and 128-bit IK is used by ME/UDM/ARPF in the following scenarios.

- To derive KAUSF in 5G AKA procedure, the concatenation of 128-bit CK and 128-bit IK (i.e. CK||IK) is used as the input key of KDF (i.e. the HMAC-SHA-256) specified in TS 33.501 [x] and TS 33.220 [y].

- To derive CK' and IK' in EAP-AKA' procedure, the concatenation of 128-bit CK and 128-bit IK (i.e. CK||IK) is used as the input key of KDF (i.e. the HMAC-SHA-256) specified in TS 33.501 [x] and TS 33.220 [y].

If MILENAGE-256-R or Tuak algorithm set is selected, the USIM/UDM/ARPF will generate 256-bit CK and 256-bit IK. However, how to enable the ME/UDM/ARPF to support 256-bit CK/IK is not clear.

5.X.2	Threats

If the mechanism to derive KAUSF/CK'/IK' with 256-bit CK/IK rather than 128-bit CK/IK is not clear, the AKA procedures cannot be completed.

5.X.3	Potential security requirements

5GS shall be able to support deriving KAUSF/CK'/IK' based on 256-bit CK and the 256-bit IK.

***************Next Change ****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 33.501 Security architecture and procedures for 5G system.

[y]	3GPP TS 33.220 Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)

***************End of the Changes ****************