3GPP TSG-SA3 Meeting #115	S3-240865

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0064 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC5 security policy for Ranging/SL positioning service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Requirements on PC5 security policy are specified for V2X services in TS 33.536 and ProSe services in TS 33.503 respectively. For Ranging/SL positioning services, PC5 security policy is also required to protect RSPP signalling messages. As RSPP signalling messages are tranferred over PC5-U, the integrity protection of PC5-U shall be required for network-based Ranging/SL positionging services. Therefore, it is proposed to add the requirements on PC5 security policy provisioning and setting for Ranging/SL positioning services in TS 33.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requirements on PC5 security policy for Ranging/SL positioning services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements on PC5 security policy for Ranging/SL positioning services are missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

6.4.2	Security requirements

The 5G system shall support mutually authentication between the UEs during unicast direct communication establishment for Ranging/SL Positioning control over RSPP.

The 5G system shall support integrity, confidentiality and anti-replay protection for the information transferred during unicast direct communication for Ranging/SL Positioning control over RSPP.

The 5G system shall support cryptographic separation for each SR5 interface and for each peer UE during unicast direct communication for Ranging/SL Positioning control over RSPP.

The 5G system shall support integrity, confidentiality and anti-replay protection for the information transferred during unicast communication for Ranging/SL Positioning control over the protocol between the UE and LMF.

The 5G system shall support a means to configure PC5 security policies to the UE for Ranging/SL positioning services.

The 5G system shall support a means to provide confidentiality, integrity and anti-replay protection of SL positioning signalling during broadcast/groupcast communication for Ranging/SL positioning.

The 5G system shall provide a means to mitigate trackability and linkability attacks of the UE during broadcast/ groupcast communication for Ranging/SL positioning.

*************** End of the Change ****************