3GPP TSG-SA3 Meeting #115 	S3-240912

# Athens, Greece, 26 February -01 March 2024

Title:	LS on Issues related Analytics context transfer between AnLF(s)

Response to:

Release:	Release 18

Work Item:	eNA_Ph3_SEC

Source:	3GPP SA3

To:	3GPP SA2

Cc:

Contact Person:

Name:	Rong Wu

E-mail Address:	Raina.wu@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 kindly ask SA2 to consider the following information.

SA3 noticed that in procedure of Transfer of analytics context and analytics subscription, as description in clause 6.1B of TS23.288, the Source AnLF may send the address (e.g. URL or FQDN) of Model file directly to the consumer(Target AnLF).

Optionally, file address(es) of the trained ML model(s), which is included only when the source NWDAF itself provides the trained ML model(s) for the analytics subscription(s) for which the related analytics context is requested.

A potential security issue is that the Target AnLF is not authorized by the NRF, since it uses Nnwdaf_AnalyticsInfo_ContextTransfer service, not the Nnwdaf_MLModelProvision service. The source AnLF could send the Model file address to the unauthorized AnLF. Regarding this, SA3 suggests that if SA2 wants Model information to be transferred, another information is needed to be transferred from source NWDAF to target NWDAF, but remove the possibility to send Model file address. If the target NWDAF then wants to get the address of the model that the source NWDAF uses, it shall use the Nnwdaf_MLModelProvision service to request the model address from the MTLF model provider.

2. Actions:

To SA2:

ACTION: 	SA2 is kindly requested to take the above information into account.

3. Date of Next TSG-SA WG3 Meetings:

SA3#115-adhoc-e	15 Apr - 19 Apr 2024	online

SA3#116		20 - 24 May 2024		Korea, KR