3GPP TSG-SA3 Meeting #115	S3-241039

- Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0062 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of 33.122 for RNAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 6.5.3.3 contains the editor's note: Editor’s Note: Whether and how the token and/or authorization request can include resource owner ID is left to stage 3. This can be removed, as the work has progressed sufficiently. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolving editor's note in 6.5.3.3, editorial in 6.5.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved editor's notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1, 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE

#### 6.5.3.1	General

The authorization function shall obtain the necessary permission from the resource owner for allowing the API invoker to access a northbound API.

RNAA shall use token-based authorization using OAuth 2.0 framework with the following roles:

-	The API invoker has the role of the OAuth 2.0 client.

-	The CCF has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA.

-	The AEF has the role of the resource server.

The access tokens used for RNAA shall contain the resource owner  ID.

The resource owner, but the resource owner ID is specified as the GPSI of the corresponding UE if the resource is related to a UE.

NOTE: The present document does not specify the resource owner.

The access token shall include the resource owner ID and the API invoker ID. The resource owner ID is GPSI . The API invoker ID binds the token to the API invoker. To avoid privacy issues, GPSI should  be different from MSISDN, SUPI etc.

Editor's Note: The details of access tokens used for RNAA need to be aligned with stage 3 (e.g., claim versus scope).

AEF shall do the authorization check of the API invocation request for accessing the resources of the resource owner. AEF checks the request against the token, including

1) checking the token integrity and

2) checking whether the GSPI (if present) in the API invocation request is compliant with the resource owner ID in the access token. As the token includes resource owner ID, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status) of the resource owner.

For OAuth flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF.

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods of clause 6.4 and clause 6.5.2 are reused.

For authorization, the following flows may be used:

-	Client credential flow (according to RFC 6749 [4]),

-	Authorization code flow (according to RFC 6749 [4]), or

-	Authorization code flow with PKCE (according to RFC 7636 [11]).

CCF shall indicate the supported flows to the API invoker.

CCF shall give service authorization which subscribers or users can use RNAA.

NOTE: In the present document, only a UE accessing its own resources is considered if the API invoker is on a UE.

# NEXT CHANGE

#### 6.5.3.3	Authorization using authorization code (optional PKCE) flow

If authorization code flow, optionally with PKCE, is used by the AEF for authorization of the API invoker, the procedures in RFC 6749 [4] and optionally RFC 7636 [11] shall be followed, with the following profile:

-	The authorization token and/or authorization request may include the resource owner ID.

NOTE: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

Editor's note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The resource owner dynamically authorizes the API invoker to access the resource owner's resources as described in RFC 6749 [4] and optionally RFC 7636 [11].

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. The access token shall contain the resource owner ID (i.e. GPSI) and the API invoker ID. If the API invoker is an AF not on a UE, the check is omitted.

Editor's Note: further details of the token are left for stage 3, this includes how to differentiate RNAA and legacy tokens

# END OF CHANGES