3GPP TSG-SA3 Meeting #115 	S3-240327

# Athens, Greece, 26 February - 1 March 2024

# Source:	KDDI Corporation

# Title:	New Key Issue on different cryptographic key lengths on AS and NAS layer

# Document for:	Approval

Agenda Item:	5.5 New Study on enabling a cryptographic algorithm transition to 256-bits

# 1	Decision/action requested

Approve the pCR to TR 33.700-41

# 2	References

[1]		SP-160464/RP-161266, "5G Architecture Options- Full Set". Joint RAN/SA Meeting, Busan, S. Korea, June 2016.

# 3	Rationale

This contribution proposes a new key issue related to different cryptographic key lengths on AS and NAS layer.

# 4	Detailed proposal

For SA3 to accept this proposal.

*** Start of 1st Change ***

## 5.X	Key Issue #X: Different cryptographic key lengths on AS and NAS layer

### 5.X.1	Key issue details

As the 5G system transitions to 256-bit cryptographic algorithms, the situation may arise that a network deployment only partially supports 256-bits cryptographic algorithms, depending on the individual operators’ deployment status and configuration policy. That is, certain network elements are already upgraded to support 256-bit cryptographic algorithms, while others do not support them, yet, or will never support them.

In these scenarios, there is a risk of different key sizes being used for AS and NAS security in a single UE session, depending on the gNB and AMF the UE is connected to. Assuming a deployment in which the AMF already supports 256-bit cryptographic algorithms, but the gNB only supports 128-bit cryptographic algorithms (or vice versa), the UE connection is protected with cryptographic keys of different length on AS and NAS layer. This example poses a challenge to the UE and the network: What is the expected behaviour in such a mixed deployment scenario?

Additionally, this issue may also occur in 5G NSA deployments. While it is expected that the 5G System will transition to 256-bit cryptography at some point in time, the same may not be true for 4G/LTE. Hence, in 5G NSA deployments the situation arises that only the Radio Access Network supports 256-bits, while the Core Network does not (i.e., option 3, 3a, 3x) or vice versa (i.e., option 4, 4a, 7, 7a, 7x) [1]. For these scenarios, too, it is necessary to specify how network and UE are to behave.

### 5.X.2	Threats

Unless gNB and AMF both support 256-bit cryptographic algorithms, AS and NAS security may be protected with cryptographic keys of different length, including legacy algorithms (Threat 3). Such discrepancies are undesirable from a security point of view.

### 5.X.3	Potential security requirements

The 5G System shall have a mechanism to avoid different key lengths being used on AS and NAS layer based on operator policy.

*** End of 1st Change ***