3GPP TSG-SA3 Meeting #115 	S3-240341

# Athens, Greece, 26 Feb - 01 March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0158 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause 6.6.3.2 – support for security procedures without network assitance check. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2023-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When a 5G ProSe UE selects security procedures without network assistance to establish a PC5 communication link, the 5G ProSe UE-to-UE relay needs to make sure it is provisionned with valid long term credentials to support the security procedures without network assistance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a check for valid long term credentials at the UE-to-UE Relay based on the Network Assistance security indicator associated with the RSC indicating that security procedures without network assistance are required. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined behaviour of UE in case of a security procedure without network assistance in the absence of valid long term credentials. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.6.3.2	Security of 5G ProSe PC5 Communication for 5G ProSe Layer-3 UE-to-UE Relay without network assistance

The security procedure in clause 6.2 is used to establish a secure PC5 link between the End UE and the 5G ProSe Layer-3 UE-to-UE Relay without network assistance with the following modifications.

-	The RSC is included in the DCR message.

-	Upon receiving the Direct Communication Request (DCR) message from the Source 5G ProSe End UE which includes an RSC and if the Network Assistance Security Indicator associated with the RSC indicates the security procedures without network assistance are required, the 5G ProSe UE-to-UE Relay needs to make sure it is provisioned with valid long term credentials, and thus, supports the security procedures without network assistance prior to initiating said security procedures. If the 5G ProSe UE-to-UE Relay does not support the security procedures without network assistance, it shall reject the Direct Communication Request message.

-	The Direct Communication Accept message is sent to the Source End UE after security establishment between the 5G ProSe Layer-3 UE-to-UE Relay and the Target End UE is successfully completed.