3GPP TSG-SA3 Meeting #115	S3-240372

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.102 | CR | 0284 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the case of a failed AUTS verification in the HE/AuC to the re-synchronisation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The case of a failed verification of the AUTS parameter by the HE/AuC is not yet specified in the re-synchronisation procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the case of a failed AUTS verification in the HE/AuC by sending an authentication failure response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this case remains unspecified, the authentication and integrity of the AUTS parameter cannot be garantied, leading to attacks that affect the user authentication and privacy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TR 33.926 CR 0087 TS 33.501 CR 1903 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 33.514 CR 0013 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ********** START OF 1st CHANGE **********

### 6.3.5	Re-synchronisation procedure

A VLR/SGSN may send two types of authentication data requests to the HE/AuC, the (regular) one described in subsection 6.3.2 and one used in case of synchronisation failures, described in this subsection.

Upon receiving a synchronisation failure message from the user, the VLR/SGSN sends an authentication data request with a "synchronisation failure indication" to the HE/AuC, together with the parameters:

-	RAND sent to the MS in the preceding user authentication request, and

-	AUTS received by the VLR/SGSN in the response to that request, as described in subsection 6.3.3.

An VLR/SGSN will not react to unsolicited "synchronisation failure indication" messages from the MS.

The VLR/SGSN does not send new user authentication requests to the user before having received the response to its authentication data request from the HE/AuC (or before it is timed out).

When the HE/AuC receives an authentication data request with a "synchronisation failure indication" it acts as follows:

1.	The HE/AuC retrieves SQNMS from Conc(SQNMS) by computing Conc(SQNMS)  f5*K(RAND).

2.	The HE/AuC verifies AUTS (cf. Subsection 6.3.3).

3.	If the verification failed, the HE/AuC sends an authentication failure response to the VLR/SGSN and it skips step 4-7.

4.	If the verification is successful the HE/AuC checks if SQNHE is in the correct range, i.e. if the next sequence number generated SQNHE using would be accepted by the USIM.

5.	If SQNHE is in the correct range then the HE/AuC continues with step (7), otherwise it continues with step (6).

6.	The HE/AuC resets the value of the counter SQNHE to SQNMS.

7.	The HE/AuC sends an authentication data response with a new batch of authentication vectors to the VLR/SGSN. If the counter SQNHE was not reset then these authentication vectors can be taken from storage, otherwise they are newly generated after resetting SQNHE. In order to reduce the real-time computation burden on the HE/AuC, the HE/AuC may also send only a single authentication vector in the latter case.

Whenever the VLR/SGSN receives a new batch of authentication vectors from the HE/AuC in an authentication data response to an authentication data request with synchronisation failure indication it deletes the old ones for that user in the VLR/SGSN.

The user may now be authenticated based on a new authentication vector from the HE/AuC. Figure 12 shows how re-synchronisation is achieved by combining a user authentication request answered by a synchronisation failure message (as described in section 6.3.3) with an authentication data request with synchronisation failure indication answered by an authentication data response (as described in this section).

Figure 12: Resynchronisation mechanism

### ********** END OF CHANGE **********