3GPP TSG-SA3 Meeting #115 	S3-240394

# Athens, GR, 26 February- 1 March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0040 | rev | 01 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add authorization procedure for Ranging/SL positioning service exposure through 5GC user plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In authorization procedure for Ranging/SL positioning service exposure defined in clause 6.3.6 in TS 33.533, there is no specification on authorization procedure for Ranging/SL positioning service exposure through 5GC user plane. Without this, SA3 specification and SA2 specification TS 23.586 are not aligned as indicated in the SA2 LS S2-2313776. This CR adds the authorization procedure for Ranging/SL positioning service exposure through 5GC user plane and adds privacy profile check for the to-be-measured UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the authorization and privacy profile check procedure for Ranging/SL positioning service exposure through 5GC user plane. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Insufficiency of the authorization procedure of Ranging/SL Positioning service exposure through 5GC user plane as specified in TS 23.586 clause 6.7.1.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.3.6.1 and 6.3.6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************Begin of the 1st Change ****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[3]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[4]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[5]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[6]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[7]	3GPP TS 38.355: " NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[8]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[9]	3GPP TS 33.303: "Proximity-based Services (ProSe); Security aspects".

[10]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[12]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[13]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

*************** End of the 1st Change ****************

***************Begin of the 2nd Change ****************

6.3.6.1	General

According to TS 23.586 [2], clause 5.6.2, Ranging/SL Positioning service can be exposed to the SL Positioning Client UE through PC5, though 5GC user plane or through 5GC control plane. The SL Positioning Client UE shall be authorized for Ranging/SL Positioning service exposure.

*************** End of the 2nd Change ****************

***************Begin of the 3rd Change ****************

6.3.6.X        Authorization procedure for Ranging/SL positioning service exposure through 5GC user plane

For Ranging/SL Positioning service exposure through the Network via user plane (i.e. clause 6.7.1.2.2 of TS 23.586 [2]), the NEF shall perform the authorization for the SL Positioning Client UE by verifying the onboarding credentials (i.e., OAuth 2.0 access token) as specified in clause 6.1 of TS 33.122 [13].

If the Client UE is not authorized, the Ranging/SL Positioning service request shall be rejected.

To preserve the privacy of the to-be-measured UEs (e.g. Target UE/SL Reference UEs), the privacy profiles of the to-be-measured UEs shall be checked. Upon receiving Ngmlc_Location_ProvideLocation Request from the NEF (i.e. step 4 in clause 6.7.1.2.2 of TS 23.586 [2]), the GMLC shall perform the privacy check for the to-be-measured UEs as defined in clause 6.3.5 during the SL-MT-LR procedure (i.e. step 5 in clause 6.7.1.2.2 of TS 23.586 [2]).

*************** End of Changes ****************