3GPP TSG-SA3 Meeting #115	S3-240445

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.203 | CR | 0277 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Validate FQDN of P-CSCF against the subjectAltName field in its server certificate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In GSMA LS S3-240259 (CVD-2023-0075 – Certificate validation on IMS access interface), GSMA has asked SA3 to update TS 33.203 to explicitly require that UE shall verify the FQDN of P-CSCF against its name in the TLS certificate (i.e., the SAN field). This is to address the MITM attack as reported by researchers in CVD-2023-0075. This CR updates TS 33.203 to implement the requests from GSMA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause O.2.1 (authentication of P-CSCF) to refer to clause O.5.2 on certificate validation and add requirement for UE to explicitly validate P-CSCF FQDN against the SAN field in P-CSCF server certificate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MITM attack as described in CVD-2023-0075. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex O.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

## O.2.1	TLS Profile for TLS based access security

When the UE and the P-CSCF implement and use TLS as specified in the present Annex O, TLS shall be implemented and used according to the TLS profile specified in TS 33.310 [24], Annex E. For all TLS versions the provisions on ciphersuites given in TS 33.310 [24], Annex E, shall apply.

-	Protection mechanisms:

-	CipherSuites with NULL encryption may be used.

-	The UE shall always include at least one CipherSuite that supports (non-NULL) encryption during the handshake phase.

NOTE 0:	TLS 1.0, TLS 1.1, and TLS 1.2 already fulfill the preceding requirement as the mandatory cipher suite has non-NULL encryption and the client includes all supported cipher suites in the Client Hello message.

-	Authentication of the P-CSCF

-	The P-CSCF shall be authenticated by the UE by presenting a valid server certificate.  The P-CSCF certificate profile shall be based on TLS certificates as presented in clause O.5.1.  UEs shall validate the P-CSCF server certificate based on clause O.5.2. UEs shall validate the FQDN of the P-CSCF against the FQDN in the subjectAltName of the P-CSCF server certificate.

-	Authentication of the UE

-	The P-CSCF shall not request a certificate in a Server Hello Message from the UE.  The HN shall authenticate the UE as specified in Annex N of this specification.

-	Verification of the TLS session endpoints

-	In order for the UE to be able to trust the TLS session endpoint, the P-CSCF certificate shall be used during the authentication procedure.

-	In order for the P-CSCF to be able to trust that the UE, which was authenticated according to Annex N, is the TLS session endpoint, the P-CSCF shall use the mechanism for associating the TLS Session ID with registration parameters IP address, port, IMPI, IMPU(s), specified in clause O.2.2, and shall have assurance that man-in-the-middle attacks can be mitigated, e.g. by following the rules in the NOTE in clause O.1.1.

*** END OF CHANGES ***