3GPP TSG-SA3 Meeting #115 	S3-240470

# Athens, Greece, 26 February - 1 March 2024

# Source:	ZTE Corporation

# Title:	New Key Issue on different cryptographic key size in EPS and 5GS interworking

# Document for:	Approval

Agenda Item:	5.5

1	Decision/action requested

It is proposed to approve the key issue.

2	References

[1]	TS 33.501 Security architecture and procedures for 5G system.

3	Rationale

This contribution is proposed to add a new key issue on different cryptographic key size in EPS and 5GS.

4	Detailed proposal

For SA3 to accept this proposal.

*** Start of 1st Change ***

5.X	Key Issue #X: Different cryptographic key size in EPS and 5GS interworking

5.X.1	Key issue details

The 5G system is expected to support 256-bit cryptographic algorithms, while 4G/LTE only supports 128-bit cryptographic algorithms. The inconsistent length of supporting cryptographic algorithms may cause problems in EPS and 5GS interworking scenarios. For example, during the handover from EPS to 5GS, the UE and the 5GS supporting 256-bit cryptographic algorithms may agree on using 128-bit cryptographic algorithms, leading to a weaker protection strength.

5.X.2	Threats

Without proper negotiation of cryptographic algorithm in EPS and 5GS interworking, the UE and the 5GS may suffer from bidding down attack, i.e., the UE and the 5GS supporting 256-bit cryptographic algorithms may agree on using 128-bit cryptographic algorithms during handover from EPS.

5.X.3	Potential security requirements

The 5G System supporting 256-bit cryptographic algorithms should be able to deal with the handover and roaming with EPS to avoid the threat of bidding down attack.

*** End of 1st Change ***