3GPP TSG-SA3 Meeting #115	S3-240479

Athens, Greece, 26th February - 1st March 2024

Title:	LS on Scope of the UDR API

Response to:

Release:	18

Work Item:	SCAS_5G_UDR, Shared_Data (C4)

Source:	SA3

To:	CT4

Cc:	-

Contact person:	Joerg Andreas

joerg.andreas@bsi.bund.de

0049 175 5557976

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	DocNumber(s) [Description e.g.. Draft TS 29.414 v0.1.0]. 
!! WARNING !! Do not insert the file directly as an object in this Word document.

# 1	Overall description

The Unified Data Repository (UDR) provides an API that delivers subscriber authentication data to the Unified Data Management (UDM). This data includes the UE’s permanent key, encPermanentKey, which serves as the anchor of trust in the entire 5G system. This discussion paper analyzes the current state of protection mechanisms for sensitive data and suggests ways to improve them. Our rationale and proposal is split into transfer and authorization security.

TS 29.505 [4] defines the use of the API to access the UE's long-term key (also called encPermanentKey in this context). The authorisation is done using OAuth2.0 according to TS 33.501 [1]. This is important because no network function other than the UDM shall have access to the UE's long-term key. This is also specified in clause 5.2.2.1 of TS 29.505 [4] as shown below.

According to the OpenAPI definition of the API endpoint (which can be seen below), the security section has three different security definitions which are “OR” connected. The first security definition ({}) indicates that security is optional (see OpenAPI Specification [3]). The next two security definitions (oAuth2ClientCredentials) require the OAuth2 Client Credentials Grant type for authentication: one with the scope nudr-dr, the other with the scope nudr-dr AND nudr-dr:subscription-data:authentication-subscription:read.

With the current specification, there is a risk that any network function can access the permanent key with the optional, hence non-security mechanism ({}), which contradicts the initial requirement of having limited access to the permanent key stored in the UDR.

# 2	Actions

To CT4

ACTION:

SA3 kindly asks CT4 about the reason why to include the non-security mechanism ({}) for accessing the permanent key and if something objects against deleting this ({}) option for access restriction.

# 3	Dates of next TSG SA WG 3 meetings

SA3#115AdHoc-e	15 - 19 April 2024	Electronic meeting

SA3#116		20 - 24 May 2024		Jeju (South Korea)

SA3#117		19 - 23 August 2024	Maastricht (Netherlands)