3GPP TSG-SA3 Meeting #115	S3-240487

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0043 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the procedure for authorization of AF/5GC NF/LCS Client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UE Ranging/SL Positioning privacy profile is under development by SA3.The existing LCS privacy check procedure covers only one target UE invovled in the LCS service request. It is unclear how to perform the privacy related check (e.g. notification and privacy verification), when one or more UEs are involved in Ranging/SL positioning service request. This paper clarifies the procedure to support notification and privacy verification defined in the UE Ranging/SL positioning privacy file reusing existing SL-MT-LR procedure, when one or more UEs are involved in Ranging/SL positioning service request. SA2 has agreed S2-2401387 which defines service operations to be used for privacy check between GMLCs from different PLMNs. This paper also proposes to align the procedure for privacy check with agreed SA2 CR. Considering the UEs may be served by different AMF, the separate Namf_Location_PrivacyNotification was introduced in addition to the existing Namf_Location_ProvidePositioningInfo service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedure for anchor GMLC to trigger privacy profile check when references UEs belonging to different PLMNs; - Add procedure for home GMLC trigger privacy verification over the introduced Namf_Location_PrivacyNotification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing privacy verification procedure for Ranging /SL Positioning service request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

6.3.5	Procedures for authorization of AF/5GC NF/LCS Client for Ranging/SL positioning service exposure

For the authorization of the AF, 5GC NF or LCS client for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If all of the UEs grant permission for Ranging/SL Positioning exposure, the GMLC shall forward the service request from the AF/5GC NF to the AMF. If none of the UEs grants permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request. If part of the UEs grant and part of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall decide t, o proceed with or reject the service request from the AF/5GC NF or LCS client based on implementation, e.g. a local rule configured by the network operator. If the GMLC decides to accept the service request, it shall only include the identities of the UEs granting permission in the service request forwarded to the AMF.

When receiving the RangingSL Pos itioning service request from the AF/5GC NF, the GMLC (i.e. anchor GMLC) interacts with the UDM to check the UE privacy profile for the UEs belonging to the same PLMN. If any of n UEs belong to different PLMNs, then the anchor GMLC sends a request to the Home GMLC of each of those UEs to check the Ranging/SL positioning privacy profile of the UE. The Home GMLCs of each of those UEs queries the UDM in its own PLMN to check the UE privacy profile and sends back the privacy profile check result to the anchor GMLC. The privacy profile check result may indicate whether the privacy check is successful or further privacy related action (e.g. notification, verification) towards the UE is required. The serving AMF address is provided to anchor GMLC if further privacy related action is required using Ngmlc_Location_PrivacyNotification_AMFaddress service. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE privacy profile for Ranging/SL positioning services.

NOTE:	The details of the UE privacy profile for Ranging/SL positioning services needs to be align with SA2.

The GMLCs (i.e. anchor GMLC, Home GMLCs) of the involved UEs triggers privacy check of these UEs towards the serving AMF(s) of these UEs via VGMLC, using SL-MT-LR message which includes indication of privacy related action for each the UEs and location type indicating "notification only". The anchor GMLC interacts with the AMF of the target UE (as UE1 in in clause 6.20.3 in TS 23.273 [3]) to request the ranging/SL positioning result of UEs per the SL-MT-LR procedure as specified in clause 6.20.3 in TS 23.273 [3], which may include an indication of a privacy related action for target UE. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message is sent to the UE by the AMF if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification (including UE notification with privacy verification) is required, the AMF shall provide failure response to the GMLC.

*************** End of the Changes ****************