3GPP TSG-SA3 Meeting #115	S3-240532

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | <CR#> | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Guidance on addressing key misalignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2023-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The KgNB may be misaligned between UE and gNB in AS SMC procedure, and this result in the integrity protection fails of AS SMC message at UE. Since the NAS SMC is success, the failure of integrity checking with AS SMC may result the whole procedure renew, the user expirence will be impacted a lot. A guidance is proposed to address this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose the UE tries to use the previous generated gNB to verify AS SMC message again. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No method to address the this, user expirence is impacted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************Start of Change*************

6.7.4.x	Guidance on addressing key misalignment

The integrity protection checking on AS SMC message may fail due to misalignment of KgNB between UE and gNB. In this case, if the UE has generated more than one KgNB, the UE may try the previously generated KgNB to verify the integrity protection of AS SMC, and take it into use if the check succeeds. Otherwise, the UE shall reply the AS SMP with the appropriate failure cause.

*************End of Change*************