3GPP TSG-SA3 Meeting #115 	S3-240539

# Athens, Greece, 26 Februray- 01 March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0187 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reformulation of verbatim copied requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA3 has conducted an activity during the SA3#111 meeting in order to remove the release numbers from the SCAS specification references and to reformulate the requirement descriptions in order to avoid verbatim content copies. However as shown below some requirements have been overlooked. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reformulations to avoid verbatim content copies from other specifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk for confusion should such the source of such verbatim text be modified in the future. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.3, 4.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Changes****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[14]	3GPP TS 33.501: "Security architecture and procedures for 5G system" (Release 16).

[15]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

**** Next Changes****

#### 4.3.6.3	Unique key values in IEs

Requirement Name: Validation of the unique key values in IEs.

Requirement Reference: 3GPP TS 29.501 Principles and Guidelines for Services Definition [13], clause 6.2.

Requirement Description: For data structures where values are accessible using names (sometimes referred to as keys), e.g. a JSON object, the name is expected to be unique. The occurrence of the same name (or key) twice within such a structure leads to an error and the rejection of the message.

Threat References: TR 33.926 [4], clause 6.3.2.2, JSON Parser not Robust

Test Case:

NOTE: This requirement can also be verified as part of Robustness and Protocol fuzzing tests as defined in clause 4.4.4 Robustness and fuzz testing according to referenced requirements.

Purpose:

Verify that the API implementation fullfills the requirements as specified in 29.501 [13], clause 6.2.

Pre-Conditions:

Test environment with network product under test. Rest of the network and network products may be simulated.

Execution Steps

1)	The test equipment sends requests with duplicate keys in message IE payload to the network product under test.

2)	The test equipment sends valid requests to network product under test

Expected Results:

1)	Network product under tests responses with an error message

2)	Network product under test still responses normally to valid requests

Expected format of evidence:

-	A testing report provided by the testing agency which will consist of the following information:

-	The used tool(s) name and version information,

-	Settings and configurations used

-	The output log file of the chosen tool that displays the results (passed/failed).

-	Test result (Passed or not)

-	Log/evidence tracing possible crashes

-	Information of any input causing unspecified, undocumented, or unexpected behaviour

**** Next Changes****

#### 4.3.6.4	The valid format and range of values for IEs

Requirement Name: Validation of the IEs limits.

Requirement Reference: 3GPP TS 29.501 Principles and Guidelines for Services Definition [13], clause 6.2

Requirement Description: The valid format and range of values for each IE, when applicable, is defined unambiguously:

-	For each message the number of leaf IEs does not exceed 16000.

-	The maximum size of the JSON body of any HTTP request does not exceed 2 million bytes.

-	The maximum nesting depth of leaves does not exceed 32.

Threat References: TR 33.926 [4], clause 6.3.2.2, JSON Parser not Robust

Test Case:

NOTE: This requirement can also be verified as part of Robustness and Protocol fuzzing tests as defined in clause 4.4.4 Robustness and fuzz testing according to referenced requirements.

Purpose:

Verify that the API implementation fullfills the requirements as specified in 29.501[13], clause 6.2.

Pre-Conditions:

Test environment with network product under test. Rest of the network may be simulated.

Execution Steps

1)	The test equipment sends requests with out of bounds IEs towards the network product under test.

Expected Results:

-	Network product under tests responses with an error message.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

-	The used tool(s) name and version information,

-	Settings and configurations used.

-	The output log file of the chosen tool that displays the results (passed/failed).

-	Test result (Passed or not).

-	Log/evidence tracing possible crashes.

-	Information of any input causing unspecified, undocumented, or unexpected behaviour.

**** End Changes****