3GPP TSG-SA3 Meeting #115 	S3-240541

# Athens, Greece, 26 Februray- 01 March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0189 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of the editor's notes in the SBA tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are currently editorr's notes remaining since the Rel-17 version and related to the indirect communication feature. These EN's were expected to be resolved back then once the stage 3 work has been completed. Unfortunately they were overlooked. Therefore, it is proposed to resolve them accordingly. The editor's note related to the CCA functionality was in fact resolved and a note stating that the test is only applicable to NFs supporting the functionality was already included in the clause body. Therefore the EN can be removed. only applicable for NF's supporting indirect communication. The editor's note on the error cases is removed and a reference to the stage-3 spec is added in the expected results. In fact, all the error messages related to CCA verification failure are described under clause 5.2.7 of TS 29.500. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution of the ediotr's notes related to the CCA test case. Addition of a new reference to TS 29.500. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved issues and hence incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Changes****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[14]	3GPP TS 33.501: "Security architecture and procedures for 5G system" (Release 16).

[15]	3GPP TS 33.2:10: "Network Domain Security (NDS); IP network layer security".

[x]	3GPP TS 29.500: "Technical Realization of Service Based Architecture".

**** Next Changes****

###### 4.2.2.2.4.1	Correct handling of client credentials assertion validation failure

Requirement Name: Correct handling of client credentials assertion validation failure

Requirement Reference: TS 33.501 [10], clause 13.3.8.3

Requirement Description:

The verification of the Client credentials assertion is performed by the receiving node, i.e., NRF or NF Service Producer in the following way:

-	It validates the signature of the JWS as described in RFC 7515 [16].

-	If validates the timestamp (iat) and/or the expiration time (exp) as specified in RFC 7519 [17].

If the receiving node is the NR	F, the NRF validates the timestamp (iat) and the expiration time (exp).

If the receiving node is the NF Service Producer, the NF service Producer validates the expiration time and it may validate the timestamp.

-	It checks that the audience claim in the the client credentials assertion matches its own type.

It verifies that the NF instance ID in the client credentials assertion matches the NF instance ID in the public key certificate used for signing the assertion.

Threat References: TR 33.926 [4], clause 6.3.x.1, Incorrect validation of client credentials assertion

NOTE: The following test case only applies if the NF under test implements verification of client credentials assertions.

Test Case:

Test Name: TC_CLIENT_CREDENTIALS_ASSERTION_VALIDATION

Purpose:

Verify that the NF under test correctly handles client credentials assertion validation failure.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a consumer NF and a SCP, which may be simulated. (Potentially simulated) consumer NF and (potentially simulated) SCP can be combined for the testing purpose.

-	The NF under test is preconfigured with the certificate of the consumer NF.

-	The NF under test is configured to require assertions for NF consumer authentication for at least one of its services.

- 	The NF under test has implemented the client credentials assertion (CCA) authentication method as specified in TS 33.501 [10], clause 13.3.8.3.

-	The tester has the private key of the consumer NF.

-	The tester has access to the interface between the consumer NF and the NF under test.

Execution Steps

Test Case 1: Failed verification of the client credentials assertion integrity

1)	The tester computes a client credentials assertion correctly, except that the signature is incorrect, and then includes the client credentials assertion in the service request sent from the consumer NF to the NF under test via the SCP.

2)	The integrity verification of the client credentials assertion by the NF under test fails.

Test Case 2: Incorrect audience claim in the client credentials assertion

1)	The tester computes a client credentials assertion correctly, except that the audience claim is incorrect, i.e., the audience claim in the client credentials assertion does not match the type of the NF under test, and then includes the signed client credentials assertion in the service request sent from the consumer NF to the NF under test via the SCP.

2)	The NF under test verifies that the audience claim in the client credentials assertion does not match its type.

Test Case 3: Expired client credentials assertion

1)	The tester computes an access token correctly, except that the expiration time (exp) has expired against the current time, and then includes the signed client credentials assertion in the service request sent from the consumer NF to the NF under test via the SCP.

2)	The NF under test verifies that the expiration time in the client credentials assertion has expired against the current time.

Expected Results:

For test cases 1~3, the NF under test rejects the consumer NF’s service request and sends back an error message according to the description under clause 5.2.7 of TS 29.500 [x].

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot containing the operational results.

**** End Changes****