3GPP TSG-SA3 Meeting #115 	S3-240555

# Athens, Greece, 26 February – 1 March 2024			                  (revision of S3-yyxxxx)

Title:	Reply LS on Roaming Hub requirements as applicable to the Modified PRINS solution

Response to:	(S3-240208) Elaborated LS reply to S3-234350 on Roaming Hub requirements as applicable to the Modified PRINS solution

Release:	Rel-18

Work Item:	Roaming5G

Source:	Huawei to be SA3

To:	SA

Cc:	SA1, SA5, CT, CT4

Contact person:	Hongyi Pu

E-mail Address:	puhongyi@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA3 received several LSes from GSMA 5GMRR related to the recent work on the modified PRINS solution. SA3 assumes that SA will coordinate a consolidated reply to GSMA. Therefore, SA3 would like to provide the following feedback on the requests included in S3-240208 on Roaming Hubs:

Answer to Q 1:

Regarding the definition of Roaming Hub, SA3 will do the alignment with the definition of Roaming Hub from GSMA in TS 33.501.

Answer to Q 2.a:

Regarding the requirement of roaming data session intervention, i.e., limiting roaming data usage, SA3 believes that it shall be implemented by the charging function of the home PLMN. Therefore, SA3 advises SA to ask GSMA to clarify the necessity of supporting Roaming Hub to intervene in the roaming data sessions. SA3 would also like to receive feedback on this from SA5 and/or SA2.

Answer to Q 2.b:

Regarding the requirement of the RH to prevent the establishment of, and to terminate the N32-c and N32-f connections, SA3 would like to clarify that preventing the establishment the N32-c and N32-f connections has been specified in clause 5.5, TS 29.573. However, considering that the RH is able to send error messages to the SEPP, SA3 believes that the termination of the N32-c and N32-f connections can be implemented by the SEPP (after receiving the error message), instead of the RH. SA3 would also like to receive feedback on this from SA2 and possibly SA1.

2	Actions

To SA:

ACTION: SA3 kindly asks SA to take into account the above replies in their reply to GSMA.

3	Dates of next TSG SA WG 3 meetings

SA3#115 Adhoc-e April 15th – 19th 2024 Online

SA3#116 May 20th – 24th 2024 Korea