3GPP TSG-SA3 Meeting #115	S3-240677

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1954 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change of requirements for DTLS over SCTP (DTLS/SCTP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CryptoSP |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The description of RFC 6083 (DTLS/SCTP) is wrong. TS 33.501 only talks about DTLS, but most of the security in RFC 6083 is provided by SCTP-AUTH (RFC 4895). DTLS replay protection is e.g., turned off. - As pointed out in LSs from RAN3, DTLS/SCTP has significant message size limitations which means that it does not work in many current networks. - As pointed out in LSs from IETF, SCTP-AUTH and DTLS/SCTP have significant security issues affecting integrity, replay protection, and availability. https://datatracker.ietf.org/meeting/115/materials/slides-115-tsvwg-sctp-auth-security-issues-00 - DTLS/SCTP relies on TLS renegotiation for rekeying, reauthentication, and transfer of new certificates. As renegotiation has been removed from DTLS 1.3, DTLS/SCTP cannot be used with DTLS 1.3 for long-time associations. 3GPP use of DTLS/SCTP requires long-time associations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The issues with DTLS over SCTP (DTLS/SCTP) (RFC 6083) is described and the requirement for support RFC 6083 for N2, Xn, F1, E1 is removed. The term DTLS is changed to DTLS/SCTP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DTLS over SCTP might be implemented and deployed even thought it has significant security and non-security issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2, 9.4, 9.8.2, 9.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ****** START OF CHANGES ****

## 9.2	Security mechanisms for the N2 interface

N2 is the reference point between the AMF and the 5G-AN. It is used, among other things, to carry NAS signalling traffic between the UE and the AMF over 3GPP and non-3GPP accesses.

The transport of control plane data over N2 shall be integrity, confidentiality and replay-protected.

In order to protect the N2 reference point, it is required to implement IPsec ESP and IKEv2 certificates-based authentication as specified in sub-clause 9.1.2 of the present document. IPsec is mandatory to implement on the gNB and the ng-eNB. On the core network side, a SEG may be used to terminate the IPsec tunnel.

In addition to IPsec, DTLS/SCTP may be supported as specified in RFC 6083 [58] to provide mutual authentication, integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [3] and the certificate profile given in clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks.

Mutual authentication shall be supported over the N2 interface between the AMF and the 5G-AN using DTLS/SCTP and/or IKEv2.

NOTE 1: 	The use of transport layer security, via DTLS/SCTP, does not rule out the use of network layer protection according to NDS/IP as specified in TS 33.210 [3]. In fact, IPsec has the advantage of providing topology hiding.

NOTE 2: 	The use of cryptographic solutions to protect N2 is an operator's decision. In case the NG-RAN node (gNB or ng-eNB) has been placed in a physically secured environment then the 'secure environment' includes other nodes and links beside the NG-RAN node.

NOTE 3: 	DTLS/SCTP as described in RFC 6083 [53] has significant message size limitations which means that it does not work in many current networks. SCTP-AUTH and DTLS/SCTP have significant security issues affecting integrity, replay protection, and availability. DTLS/SCTP can only be used with the obsolete DTLS 1.2 and cannot be used with DTLS 1.3.

## ****** NEXT CHANGE ****

## 9.4	Security mechanisms for the Xn interface

Xn is the interface connecting NG-RAN nodes. It consists of Xn-C and Xn-U. Xn-C is used to carry signalling and Xn-U user plane data.

The transport of control plane data and user data over Xn shall be integrity, confidentiality and replay-protected.

In order to protect the traffic on the Xn reference point, it is required to implement IPsec ESP and IKEv2 certificate- based authentication as specified in sub-clause 9.1.2 of the present document with confidentiality, integrity and replay protection. IPsec shall be supported on the gNB and ng-eNB.

In addition to IPsec, for the Xn-C interface, DTLS/SCTP may be supported as specified in RFC 6083 [58] to provide mutual authentication, integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [3] and the certificate profile given in clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks.

Mutual authentication shall be supported over the Xn interface between the NG-RAN nodes using DTLS/SCTP and/or IKEv2.

NOTE 1: 	The use of transport layer security, via DTLS/SCTP, does not rule out the use of network layer protection according to NDS/IP as specified in TS 33.210 [3]. In fact, IPsec has the advantage of providing topology hiding..

NOTE 2: 	The use of cryptographic solutions to protect Xn is an operator's decision. In case the NG-RAN node (gNB or ng-eNB) has been placed in a physically secured environment then the 'secure environment' includes other nodes and links beside the NG-RAN node.

NOTE 3: 	DTLS/SCTP as described in RFC 6083 [53] has significant message size limitations which means that it does not work in many current networks. SCTP-AUTH and DTLS/SCTP have significant security issues affecting integrity, replay protection, and availability. DTLS/SCTP can only be used with the obsolete DTLS 1.2 and cannot be used with DTLS 1.3.

QoS related aspects are further described in sub-clause 9.1.3 of the present document.

## ****** NEXT CHANGE ****

### 9.8.2	Security mechanisms for the F1 interface

The F1 interface connects the gNB-CU to the gNB-DU. It consists of the F1-C for control plane and the F1-U for the user plane. The security mechanisms for the F1 interface connecting the IAB-node to the IAB-donor-CU are detailed in clause M.3.3 of this document.

In order to protect the traffic on the F1-U interface, IPsec ESP and IKEv2 certificates-based authentication shall be supported as specified in sub-clause 9.1.2 of the present document with confidentiality, integrity and replay protection.

In order to protect the traffic on the F1-C interface, IPsec ESP and IKEv2 certificates-based authentication shall be supported as specified in sub-clause 9.1.2 of the present document with confidentiality, integrity and replay protection.

IPsec is mandatory to implement on the gNB-DU and on the gNB-CU. On the gNB-CU side, a SEG may be used to terminate the IPsec tunnel.

In addition to IPsec, for the F1-C interface, DTLS/SCTP may be supported as specified in RFC 6083 [58] to provide mutual authentication, integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [3] and the certificate profile given in clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks..

Mutual authentication shall be supported over the F1-C interface between the gNB-CU and the gNB-DU using DTLS/SCTP and/or IKEv2.

NOTE 1: 	The use of transport layer security, via DTLS/SCTP, does not rule out the use of network layer protection according to NDS/IP as specified in TS 33.210 [3]. In fact, IPsec has the advantage of providing topology hiding.

NOTE 2: 	The use of cryptographic solutions to protect F1 is an operator's decision. In case the gNB or the IAB-node has been placed in a physically secured environment then the 'secure environment' includes other nodes and links beside the gNB or the IAB-node.

NOTE 3: 	The security considerations for DTLS over SCTP are documented in RFC 6083 [58].

NOTE 4:	The support of DTLS/SCTP (with mutual authentication) for F1-C, between the IAB-node (gNB-DU) and the IAB-donor-CU, is optional for the IAB-node and the IAB-donor-CU.

NOTE 5: 	DTLS/SCTP as described in RFC 6083 [53] has significant message size limitations which means that it does not work in many current networks. SCTP-AUTH and DTLS/SCTP have significant security issues affecting integrity, replay protection, and availability. DTLS/SCTP can only be used with the obsolete DTLS 1.2 and cannot be used with DTLS 1.3.

## ****** NEXT CHANGE ****

### 9.8.3	Security mechanisms for the E1 interface

The E1 interface connects the gNB-CU-CP to the gNB-CU-UP. It is only used for the transport of signalling data.

In order to protect the traffic on the E1 interface, IPsec ESP and IKEv2 certificates-based authentication shall be supported as specified in sub-clause 9.1.2 of the present document with confidentiality, integrity and replay protection.

In addition to IPsec, DTLS/SCTP may be supported as specified in RFC 6083 [58] to provide mutual authentication, integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [3] and the certificate profile given in clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks.

Mutual authentication shall be supported over the E1interface between the gNB-CU-CP and the gNB-CU-UP using DTLS/SCTP and/or IKEv2.

IPsec is mandatory to support on the gNB-CU-UP and the gNB-CU-CP. Observe that on both the gNB-CU-CP and the gNB-CU-UP sides, a SEG may be used to terminate the IPsec tunnel.

NOTE 1: 	The use of transport layer security, via DTLS/SCTP, does not rule out the use of network layer protection according to NDS/IP as specified in TS 33.210 [3]. In fact, IPsec has the advantage of providing topology hiding.

NOTE 2: 	The use of cryptographic solutions to protect E1 is an operator's decision. In case the gNB has been placed in a physically secured environment then the 'secure environment' includes other nodes and links beside the gNB.

NOTE 3: 	DTLS/SCTP as described in RFC 6083 [53] has significant message size limitations which means that it does not work in many current networks. SCTP-AUTH and DTLS/SCTP have significant security issues affecting integrity, replay protection, and availability. DTLS/SCTP can only be used with the obsolete DTLS 1.2 and cannot be used with DTLS 1.3.

## ****** END OF CHANGES ****