3GPP TSG-SA3 Meeting #115	S3-240744

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCPAC: FC values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The LS from last SA3 meeting to RAN2/RAN3 clarified that SA3 use the same counter (SN Counter) for KSN derivations in SCPAC that is already used for DC KSN derivations. While it intutively may not be any immediate risks for the same counter values being used twice, resulting in the same keys being derived for two different base stations, there is a risk that some corner case has been overlooked, or that future extenstions and optimizations make that happen. Therefore, to be on the safe side, and to be in line with the recommendations in TS 33.220 to use a different FC value for each purpose, this pCR propsoses to resolve the editor’s note in annex A.16 by using a separate FC value when deriving keys for SCPAC use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new clause with a new FC value for SCPAC KSN derivations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk that same key is derived for more than one purpose, leading to loss of confidentiality and integrity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.16, A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.16	Derivation of KSN for Dual Connectivity

This input string is used when the MN and UE derive KSN. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

# * * * * Start of Second change * * * *

# A.x	Derivation of KSN for Subsequent CPAC

To ensure that KSNs derived for Dual connectivity do not accidentally collide with KSNs derived for Subsequent CPAC, the two derivations use different FC values.

This input string is used when the MN and UE derive KSN during dual connectivity for Subsequent CPAC. The following input parameters shall be used:

-	FC =0xYY,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KgNB.

# * * * * End of changes * * * *