3GPP TSG-SA3 Meeting #115	S3-240747

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCPAC: Algorithm negotiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Algorithm negotiation for subsequent CPAC (SCPAC) needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the algorithm negotiation for subsequent CPAC (SCPAC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The algorithm negotiation for the subsequent CPAC (SCPAC) procedure is not clarified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.10.2.x.y	Negotiation of security algorithms for SCPAC

The MN shall receive the UE security capabilities from the AMF or the previous NG-RAN node. These security capabilities include both NR security capabilities.

When establishing one or more DRBs for a UE at the SN, as shown on Figure 6.10.2.x.x-1, the MN shall provide the UE security capabilities of the UE to the candidate Target SN in the SN Addition/Modification Request message.

Upon receipt of this message, the candidate Target SN shall select the algorithms with highest priority in its locally configured list of algorithms that are also present in the received UE security capabilities and include the selected algorithms in the SCG configuration in the SN Addition/Modification Request Acknowledge to the MN.

The Target SN shall select one encryption algorithm and one integrity protection algorithm, which associates to all SN Counter values in the sequence of distinct SN Counter values which were assigned by the MN to the Target SN.

The MN shall provide the selected algorithms to the UE for all candidate SCG configuration(s) for one or multiple candidate Target SN(s) during the RRCReconfiguration procedure, that configures the DRBs with the candidate Target SN for the UE. When the UE selects a Target SN for the first time or subsequent time, then the UE shall use the indicated algorithms, received in the SCG configuration for the Target SN, for the DRBs whose PDCP terminates on the Target SN.

NOTE: The algorithms that the UE uses with the MN can be the same or different to the algorithms used with the Target SN.

# * * * * End of changes * * * *