3GPP TSG-SA3 Meeting #115	S3-240778

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0059 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the authorization for UEs belonging to different PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in TS 23.273, inter-PLMN operation is supported for Ranging services, e.g. the Target UE is subscribed in PLMN A and the Reference UE is subscribed in PLMN B. It was concluded in SA2#160 e-meeting that the new Ngmlc_Location_PrivacyCheck_IDMapping service operation is added to perform the privacy checking and ID resolution for UEs belonging to different PLMN(s). Therefore, it is proposed to add NL3 as a reference point which supports Ranging/SL positioning security architecture. According to the authorization procedure for Ranging service exposure defined in clause 6.3.5, all the involved UEs need to be authorized for protecting the privacy of the UEs, i.e. the privacy profiles of the UEs in potentially different PLMNs need to be checked. However, how to authorize the UEs belonging to other PLMNs is not clearly specified in clause 6.3.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the NL3 interface in clause 4.2.2. - Added the description for the authorization of UEs belongling to different PLMNs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The GMLC in one PLMN cannot check the authorization of all the involved UEs in case the UEs belong to different PLMNs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

4.2.2	Reference points

In addition to the reference points specified in clause 4.2 of TS 23.586 [2], the following reference points are added or enhanced for supporting Ranging/SL positioning security architecture:

NL3:	The reference point between the GMLCs in different PLMNs. It is used to transport the UE authorization result for authorization of Ranging/SL Positioning service exposure.

NL6:	The reference point between the UDM and the GMLC. It is used to transport the UE privacy profile to GMLC for authorization of Ranging/SL Positioning service exposure.

PC8*:	The reference point between the UE and the SLPKMF, which relies on 5GC user plane for transport (i.e. an "over IP" reference point). It is used to transport security materials to UEs for Ranging/SL Positioning discovery and communication.

Npc9*:	The reference point between the SLPKMFs of the UEs subscribed in different PLMNs. It is used to transport security materials between SLPKMFs to support Ranging/SL Positioning services in inter-PLMN scenarios.

Npc10*:	The reference point between the UDM and the SLPKMF. It is used to request subscription information related to Ranging/SL Positioning service from the UDM for UE authorization.

*************** Start of the 2nd Change ****************

6.3.5	Procedures for authorization of AF/5GC NF/LCS Client for Ranging/SL positioning service exposure

For the authorization of the AF, 5GC NF or LCS client for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If all of the UEs grant permission for Ranging/SL Positioning exposure, the GMLC shall forward the service request from the AF,5GC NF or LCS client to the AMF. If none of the UEs grants permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request. If part of the UEs grant and part of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall decide to proceed with or reject the service request from the AF, 5GC NF or LCS client based on implementation, e.g. a local rule configured by the network operator. If the GMLC decides to accept the service request, it shall only include the identities of the UEs granting permission in the service request forwarded to the AMF.

When receiving the Ranging/SL Positioning service request from the AF,5GC NF or LCS client, the GMLC (i.e. anchor GMLC) interacts with the UDM to check the UE privacy profile for the UEs belonging to the same PLMN. If any of the n UEs belongs to different PLMNs, the anchor GMLC shall trigger privacy check toward the Home GMLC(s) of each of those UEs via NL3. The Home GMLC(s) queries the UDM(s) in its own PLMN to check the UE Ranging/SL Positioning privacy profile and sends back the privacy check result to the anchor GMLC. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE privacy profile for Ranging/SL positioning services.

NOTE:	The details of the UE privacy profile for Ranging/SL positioning services needs to be align with SA2.

NOTE:	The address of the Home GMLC of the UE(s) in a different PLMN is determined by the anchor GMLC based on local configuration or by the NRF query.

When checking UE Ranging/SL Positioning privacy profiles, if one or more profiles include the indicator of privacy check related action, the Home GMLC of the UE(s) shall retrieve the serving AMF(s) from the UDM of the UE(s) and trigger privacy check of the UE(s) towards the serving AMF(s), using Namf_Location_ProvidePositioningInfo message which includes the indicator(s) of privacy check related action for the UE(s) and location type indicating "notification only". If the indicator(s) of privacy check related action indicates that the UE(s) must either be notified or notified with privacy verification, a notification invoke message is sent by the serving AMF(s) to the UE(s) if the signalling connection established. The serving AMF(s) shall respond to the Home GMLC of the UE(s) with privacy check result(s) of the UE(s). If the Ranging/SL Positioning service exposure is disallowed by any of the UE(s), or signalling connection establishment fails for UE notification (including UE notification with privacy verification), the serving AMF(s) shall include the failure cause(s) of the UE(s) in the response message to the Home GMLC. The Home GMLC sends to the anchor GMLC the privacy check result of the UEs which includes only the identities of the UEs allowing service exposure.

*************** End of the Changes ****************