3GPP TSG-SA3 Meeting #115 	S3-240787

# Athens, Greece, 26 February - 1 March 2024

Title:	[Draft] Reply LS to reply LS on Decorated NAI format for 5G-NSWO for SNPN Scenarios

Response to:	Reply LS on Decorated NAI format for 5G-NSWO for SNPN Scenarios (S3-240219)

Release:	Rel-18

Work Item:	eNPN_Ph2

Source:	Xiaomi [to be SA3]

To:	CT4

Cc:	SA2

Contact person:	Henry(Haoran) Liang

lianghaoran at xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	n/a

1	Overall description

SA3 would like to thank CT4 for their LS C4-235479 on decorated NAI.

Refering to clause 5.30.2.9.3 of TS 23.501, the Credentials Holder can be built on PLMN/SNPN. Clause 5.30.2.9.2 of TS 23.501 indicates that the Credentials Holder can also be built on AAA server.

Figure 1 and Figure 2 are captured from clause 4.2.15 of TS 23.501. These figures show that how AAA server based Credentials Holder can be used to support NSWO.

Figure 1: Reference architecture to support authentication for Non-seamless WLAN offload using credentials from Credentials Holder using AAA Server

Figure 2: Reference architecture to support authentication for Non-seamless WLAN offload using credentials from Credentials Holder using AAA Server via 5GC

The decorated NAI defined in the attached C4-235478 only considers the Credentials Holder that is built in PLMN/SNPN.

Since the decorated NAI for AAA server based Credentials Holder is still not clear, the SNPN cannot correctly route the decorated NAI to the AAA server to complete the authentication procedure.

2	Actions

To CT4

ACTION: 	SA3 kindly asks CT4 to take the above into consideration.

3	Dates of next TSG SA WG 3 meetings

SA3#115-adhoc-e15 April 2024 -19 April 2024	TBD, TBD

SA3#116	 20 May 2024 - 24 May 2024	TBD, Korea