3GPP TSG-SA3 Meeting #115	S3-240803

Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0065 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding notes for Ranging/SL positioning broadcast/groupcast communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to Revised WID on Expanded and Improved NR Positioning in RP-232670 approved at RAN#101, RAN specification on the protocol and procedures for SL positioning between UEs (SLPP) is restricted to unicast session-based signalling procedure only in Release 18. It means that SLPP broadcast and groupcast signalling procedures are not supported in Release 18. However, security requirements and procedures for Ranging/SL positioning broadcast/groupcast communication have already been specified in current release of TS 33.533. In order to align with the updated scope of features supported by RAN WGs, it is proposed to add notes in corresponding clauses of TS 33.533 indicating that security for Ranging/SL positioning broadcast/groupcast communication is not applicable in this release of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the notes in clauses 6.4.2 and 6.4.4.1 indicating that security for Ranging/SL positioning broadcast/groupcast communication is not applicable in this release of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security requrements and procedures are specified for the features not supported in Release 18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2, 6.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

6.4.2	Security requirements

The 5G system shall support mutually authentication between the UEs during unicast direct communication establishment for Ranging/SL Positioning control over RSPP.

The 5G system shall support integrity, confidentiality and anti-replay protection for the information transferred during unicast direct communication for Ranging/SL Positioning control over RSPP.

The 5G system shall support cryptographic separation for each SR5 interface and for each peer UE during unicast direct communication for Ranging/SL Positioning control over RSPP.

The 5G system shall support integrity, confidentiality and anti-replay protection for the information transferred during unicast communication for Ranging/SL Positioning control over the protocol between the UE and LMF.

The 5G system shall support a means to provide confidentiality, integrity and anti-replay protection of SL positioning signalling during broadcast/groupcast communication for Ranging/SL positioning.

The 5G system shall provide a means to mitigate trackability and linkability attacks of the UE during broadcast/ groupcast communication for Ranging/SL positioning.

NOTE:	Security requirements for Ranging/SL positioning broadcast/groupcast communication are not applicable in this release of the specification.

*************** Start of the 2nd Change ****************

6.4.4.1	General

This clause describes the security mechanism for broadcast/groupcast communication over RSPP. The RSPP messages for broadcast/groupcast communication are protected at the RSPP layer.

NOTE:	Security mechanism for Ranging/SL positioning broadcast/groupcast communication over RSPP is not applicable in this release of the specification.

*************** End of the Changes ****************