3GPP TSG-SA3 Meeting #117	S3-242763

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0212 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MCXSec4] 33180 R19 MC Recording Server Introduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus, Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCXSec4 |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introducing MC Recording server |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the MC Recording server for collecting and storing user media. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This CR is linked to S6-243037 document. These two CRs should be agreed together. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2, 10.2.3 and 10.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

### 10.2.2	Collection of user media

It is assumed that collected Mission Critical media is held in its encrypted form within mass data storage.   The storage solution is out-of-scope of this document.

User media is collected from the media paths within the MC Domain. It is expected that the encrypted media required to be collected (MCPTT, MCData, MCVideo or MC Location) shall be routed from the appropriate MC Service Server to the MC Recording server.

Where SDS messages are routed within a signalling path, media will need to be extracted from within MCData signalling paths by the MCData Server. It is expected that the encrypted media routed over the signalling path shall be routed from the MCData Server to the MC Recording server.

To identify and process the collected and encrypted user media, user event logs associated with the media are required, as defined in Clause 10.1.2.

### 10.2.3	Storing of user media

User media in the MC System is end-to-end encrypted by default. Consequently, media can be recorded without modification and without additional protection, or by decrypting it on the MC Recording server and re-encrypting it before storing it to mass data storage, if the level of security provided by other means is meeting the operator's security policy. The re-encryption method is out of scope of the current specification. Media should be recorded alongside:

-	a unique identifier for the collected media

-	a unique identifier for a user event (with which the media is associated).

NOTE:	Collected associated user event metadata may or may not be stored with the media.

### 10.2.4	Decryption of user media

Editor's Note:  Updates to this section are FFS.

To decrypt a specific target user's media for audit or discreet listening at a specific time 'T', the following process should be used for decryption of user media.  The controlling entity shall be either the KMS or a secured logging device.

1.	The auditor obtains the target user's key material and KMS certificate that was active at time 'T'. This could be performed, for example, using an Audit Client. An Audit Client is a Key Management Client with the special access privilege to request previously-issued user key material for existing clients  The provision of user key material by the KMS grants the auditor access to the user’s communication content..

a.	Any request made by the auditor shall be controlled and logged(to allow the audit action to be audited) .

b.	It is recommended that each release of key material be authorised by a secondary auditor (e.g. a double-lock mechanism).

c.	It is recommended that the number of requests from each auditor within a time-period be limited. It is also recommended that the total number of requests from all auditors within a time-period be limited.

NOTE:	The release of key material for a user at time 'T' only allows the auditor access to content for the  defined 'key period' associated to time ‘T’. By limiting the total number of requests (e.g. to 0.1% of users), this limits the auditor’s access to communications. These controls help to ensure that the granted access to user content is time-limited and proportionate.

2.	The auditor extracts the user events associated with the user at time 'T'.

3.	The auditor extracts the MIKEY messages from the signalling events and use the audited user's KMS-supplied key material to decrypt the media encryption keys held within the MIKEY messages.

4.	The auditor is now able to associate media with user events and use the media encryption keys extracted from MIKEY message to decrypt the media.

***	END OF CHANGES	***