3GPP TSG-SA3 Meeting #117	S3-242786

Maastricht, Netherlands, 19th - 23th August 2024

# Source:	KDDI, CableLabs, Ericsson

# Title:	Annexes on 256-bit security and backwards compatibility

# Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

It is proposed to add the following Annex to TR 33.700-41[1].

# 2	References

[1]	3GPP TR 33.700-41 "Study on enabling a cryptographic algorithm transition to 256-bits"

[2]	S3‑242449 “Analysis of backwards compatibility”

# 3	Rationale

In the last meeting, SA3 discussed the addition of an informative Annex to TR 33.700-41, focused on an analysis of backwards compatibility of 256-bit cryptographic algorithms [2]. This contribution did not get agreed.

Although SA3 is set to close TR 33.700-41 during SA3#117, KDDI believes that the information contained in the above-mentioned contribution would be a valuable addition for several reasons. Firstly, is summarizes the SA3 discussions throughout this study. Secondly, it shows that SA3 did their due diligence on this 256-bit transition by recapping our discussion. And thirdly, it provides informative recommendations to operators on how to manage specific aspects of the 256-bit transition.

The proposed changes below are based on S3‑242449, incorporating further changes based on offline discussion since the last meeting.

# 4	Detailed proposal

***START OF 1st CHANGE***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[XX] 	UK NCSC, "Next steps in preparing for post-quantum cryptography"
https://www.ncsc.gov.uk/whitepaper/next-steps-preparing-for-post-quantum-cryptography

***END OF 1st CHANGE***

***START of 2nd CHANGE***

# Annex X (Informative): Considerations on 256-bit security

One aspect of using algorithms with larger key size is to improve security i.e. getting approximately 256-bit of security instead of the approximately 128-bit of security.

NOTE: Although an algorithm can be proven to have x-bit of security, it is more difficult to say that an entire system has an exact level of security. Hence the usage of "approximately" above.

Since 128-bit key algorithms are still considered as being secure [XX], there is no immediate need to replace them with 256-bit variants in the 5G system for security purposes. Moreover, simply replacing the encryption algorithms with 256-bit ones does not ensure approximately 256-bit security for the entire system as algorithms are only one of many factors which decide the security level of the entire system. Rather, the introduction of 256-bit algorithms should be understood as an enhancement driven by the need of increased throughput and explicitly not a need to repair a security vulnerability in the current 3GPP system. The approach taken in this study is to take it stepwise and study how 256-bit algorithms can coexist with 128-bit algorithms in 5G. However, this does not mean that the target is to achieve approximate 256-bit security for the entire 5G system when the new algorithms are used. Since parts of the system are still using less than 256-bit security, the overall security will not reach 256-bit levels. For example, even though air-interface integrity protection may use a 256-bit key algorithm, the resulting MAC is still truncated to 32-bits.

It was decided that 256-bit key algorithms were to be added as new algorithms and that the resulting difference in the overall security level of the 5G system does not warrant changes to the specification at this point. Therefore, 256-bit key algorithms can be used together with the existing ones in 5G.

***END OF 2nd CHANGE***

***START of 3rd CHANGE***

# Annex Y (Informative): Analysis of backwards compatibility

# Y.1 	Introduction

Introduction of any new algorithm into the 5G system will have a certain amount of impact. The deployment of the new algorithms will not take place everywhere all at once. Some deployments will not support the new algorithms at all. This is nothing new.

Table Y.1-1 summarizes the possible scenarios of mixed cryptographic capabilities/policies, assuming that supporting/using only 256-bit crypto algorithms is allowed. It classifies scenarios into four categories, including authentication and key agreement (AKA), access security (AS), non-access security (NAS), and key derivation (KD). The three possible capabilities (128-bit only, 128-bit and 256-bit, and 256-bit only) are numbered by 1, 2, and 3 respectively. The label of a scenario is denoted as: <Category><UE capability>.<Network capability>. For example, AKA1.2 represents the case of authentication and key agreement where USIM/UE supports only 128-bit crypto algorithm, and network function (AUSF/UDM) supports both 128-bit and 256-bit crypto algorithms.

NOTE: it is assumed that AUSF and UDM will be upgraded at the same time and share the same crypto capabilities.

Table Y.1-1 Scenarios of mixed crypto capabilities

|  |  | gNB |  |  | AMF/SEAF |  |  | AUSF/UDM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 128-bit | 128-bit &256-bit | 256-bit | 128-bit | 128-bit &256-bit | 256-bit | 128-bit | 128-bit &256-bit | 256-bit |
| USIM | 128-bit | N/A | N/A | N/A | N/A | N/A | N/A | AKA1.1 | AKA1.2 | N/A |
|  | 128-bit &256-bit | N/A | N/A | N/A | N/A | N/A | N/A | AKA2.1 | AKA2.2 | AKA2.3 |
|  | 256-bit | N/A | N/A | N/A | N/A | N/A | N/A | N/A | AKA3.2 | AKA3.3 |
| ME | 128-bit | AS1.1 | AS1.2 | AS1.3 | NAS1.1 | NAS1.2 | NAS1.3 | KD1.1 | KD1.2 | KD1.3 |
|  | 128-bit &256-bit | AS2.1 | AS2.2 | AS2.3 | NAS2.1 | NAS1.2 | NAS2.3 | KD2.1 | KD2.2 | KD2.3 |
|  | 256-bit | AS3.1 | AS3.2 | AS3.3 | NAS3.1 | NAS1.3 | NAS3.3 | KD3.1 | KD3.2 | KD3.3 |

NOTE: UE consists of the USIM and ME, hence the UE’s capabilities is inferable from the table above.

There are already today a set of algorithms specified for 5G (128-NEA-1,2,3 and 128-NIA-1,2,3), where not all systems support all algorithms (only 1 and 2 are required). Still 5G works because it was designed with this in mind. For AS and NAS protocols there are negotiation mechanisms in place that will help both peers of a connection to find a set of algorithms (one for confidentiality and one for integrity) that they share. The network operator is in control of the prioritization of the algorithms. The algorithm with the highest priority that both endpoints can agree on gets selected.

If the introduction of 256-bit key algorithms is considered as introduction of any new algorithms, it is a fair assumption that current mechanisms can be reused and provide equally strong negotiation protection for the 256-bit algorithms just by adding possibility to negotiate the new algorithms (i.e. define new algorithm identifiers/code points). In fact, if this was not the case, there would be a vulnerability in the existing algorithm negotiation process. Moreover, adoption of 256-bit algorithms does not affect the security of current 128-bit algorithms, the existing mechanisms for algorithm negotiation can be reused with no identified security concern.

The purpose of this Annex is to discuss the different aspects of introducing 256-bit algorithms and its potential impact to the 5G system, especially in terms of backward compatibility when 128-bit and 256-bit algorithms are co-existed in the 5G system.

The current mechanisms allow for introducing new algorithms. That would work as described below. It does not ensure 256-bit security for the system as discussed in Annex X.

As summarised in the Table Y.1-1, AKA, AS, NAS and KD need to be considered on the introduction of 256-bit algorithms. For AKA, RES* and XRES* derivation function and HRES* and HXRES* derivation function are defined in the Annex A of TS 33.501 [3], and both (X)RES* and H(X)RES* are defined as the 128 least significant bits of the output of the KDF, namely SHA-256. Therefore, AKA is independent of the introduction of 256-bit algorithms, and not covered in the present document. The impact to key derivation and AS/NAS protocols will be described in clauses Y.2 and Y.3. For more detailed analysis of the impact to AS protocols, the impact to dual connectivity, RRC-reconnection, and handovers and interworking will be discussed in clauses Y.4, Y.5 and Y.6.

# Y.2 	Long-term key and key hierarchy

The Key (or long-term key(s) of the subscription credential(s)) can be either 128 or 256 bits, see clause 6.3.7 of TS 33.501 [2]. If a 128-bit long-term key is used for authenticating the UE, the resulting keys (CK, IK) are both 128-bits. These keys are always used together (CK || IK) as the input for the key derivation function (KDF) defined in Annex A.2 of TS 33.501 [3]. The KDF outputs a 256-bit KAUSF K key. All subsequent keys in the key hierarchy are 256-bit long and compatible with 256-bit algorithms (clause 6.2 or TS 33.501 [2])

As mentioned above, all output keys in the 5G key hierarchy are 256 bits, which means that they need truncation to be used in 128-bit algorithms. The truncation is defined in Annex A.8 of TS 33.501 [2] as:

For an algorithm key of length n bits, where n is less or equal to 256, the n least significant bits of the 256 bits of the KDF output shall be used as the algorithm key.

According to the above, when n=128, the 128 least significant bits of the key are used, and when n=256, all of the bits are used.

It was confirmed that the existing key hierarchy and truncation work for both 128-bit and 256-bit algorithms without any changes to the mechanisms.

NOTE: The truncation step may need to be updated in the figures of TS 33.501.

# Y.3	Impact on AS and NAS protocols

For AS/NAS security algorithm selection, AS/NAS security mode commands are used to indicate the selected ciphering and integrity protection algorithms to the UE. AS/NAS security mode commands are independent of the key length and can be used for both 128-bit and 256-bit algorithms.

Clause 6.4.6 of TS 33.501 [2] defines the protection of initial NAS message, and one of the information elements in the initial NAS message is UE security capabilities. The UE security capabilities contain the list of algorithms which UE supports, and the key length of the algorithms is not involved. Therefore, the initial NAS message can carry the list of supported 256-bit algorithms if algorithm identifier values are assigned.

In clause 6.7.2 of TS 33.501 [2], the information element for NAS Security Mode Command is defined. Mandatory elements are the replayed UE security capabilities, the selected NAS algorithms, and the ngKSI for identifying the KAMF. Here, the replayed UE security capabilities contain algorithms which the UE support and selected NAS algorithms are the selected algorithms. The key length is not involved in this procedure, and NAS Security Mode Command can be applied for 256-bit algorithms if corresponding algorithm identifier values are assigned to 256-bit algorithms.

The AS algorithms are selected between UE and gNB in the similar manner as NAS algorithms. Therefore, 256-bit AS algorithms can be adopted to AS algorithm selection.

It was agreed that 128-bit algorithms are still sufficiently secure for the 5G system and that no need to differentiate the security levels provided by128-bit and 256-bit algorithms. Therefore, from a specification perspective, 256-bit algorithms can be treated as an additional set of algorithms and the NAS and AS SMC procedures for negotiating algorithms between the UE and AMF and the UE and gNB respectively can be used without any changes.

What is needed is for the new algorithms to be assigned new identifiers in clause 5.11.1 of TS 33.501 [2] and that new code points are assigned in the UE security capability information element for the new encryption and integrity protection algorithms in stage 3 specifications.

# Y.4	Impact on Dual connectivity

In NR-NR Dual Connectivity (NR-DC) the UE is simultaneously connected to more than one RAN node, a MN and a SN, see clause 6.10 of TS 33.501 [2].

The rules for setting up security contexts in Dual connectivity are very complex, but (very much simplified), it can be summarized so that if protection on the air interface is activated on one access, it needs to be active on the other access as well. There is no rule that the two accesses need to use the same air-interface security algorithms, see the NOTE in clause 6.10.3.3 of TS 33.501 [2].

It was confirmed that 128-bit algorithms are secure for the 5G system and that 256-bit algorithms are just a set of additional algorithms. Therefore, it is possible to use a 128-bit algorithm over one access and a 256-bit algorithm over the other access in Dual connectivity, reusing existing mechanisms.

# Y.5	Impact on RRC Connection Re-establishment

When RRC connection is re-established, the same air-interface security algorithm as the source gNB can be selected between the UE and target gNB. However, this does not prohibit the algorithm negotiation between the UE and the target gNB, a new algorithm can be selected based on the UE security capability and the security policy of gNB.

When the new algorithm is to be selected between the UE and the target gNB, AS algorithm selection can be used. As discussed in Y.3, AS algorithm selection can be used for both 128-bit and 256-bit algorithms if adequate algorithm identifier values are assigned.

It was confirmed that 128-bit algorithms are secure for the 5G system and that 256-bit algorithms are just a set of additional algorithms, then it is possible to use a 256-bit algorithm over one access and handover to an access that is using a 128-bit algorithm or vice versa.

# Y.6	Impact on Handovers and Interworking

After a handover from one network that does not support the 256-bit air-interface security algorithms (e.g. a 4G network) to a network supporting one or more of the 256-bit algorithms, the AMF of the latter can run a new NAS SMC with UE to switch to the 256-bit algorithm. Hence, the UE does not "get stuck" in using the old type of algorithms, but the operator policy can take the UE's full UE security capability into account.

As discussed in Y.3, NAS SMC can support both 128-bit and 256-bit algorithms if adequate algorithm identifier values are assigned.

It was confirmed that 128-bit algorithms are secure for the 5G system and that 256-bit algorithms are just a set of additional algorithms, then it is possible to use a 256-bit algorithm over one access and handover to an access that is using a 128-bit algorithm or vice versa. Therefore, there is no security issue identified for reusing the existing mechanisms in handovers and interworking scenarios.

***END OF 3rd CHANGES***