3GPP TSG-SA3 Meeting #117 	S3-242825

# Maastricht, Netherlands 19 - 23 August 2024

# Source:	InterDigital

# Title:	A solution for Key Issue #2.1: Secure retrieval of 5G system UE IDs.

# Document for:	Approval

# Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to approve the pCR to TR 33.749.

# 2	References

[1]	3GPP TR 33.749: "Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3"

# 3	Rationale

This contribution proposes a solution for the Key Issue #2.1: Secure retrieval of 5G system UE Ids and privacy related information in [1].

# 4	Detailed proposal

Approve the following changes to TR 33.749 [1].

*** Start of Change ***

## 6.X	Solution #X: Authorizing the Service Consumer when Resolving an IP Address to a UE ID

### 6.X.1	Solution overview

The Nbsf_Management_Discovery service is used to resolve a UE IP Address to a SUPI.  The solution describes how to enhance the service so that the BSF can verify that the consumer is authorized to obtain the UE ID.

### 6.X.2	Solution details

This solution supports the 5G System to verify the IP address in the request to obtain a UE Identifier. In the proposed procedure, both the EEC and BSF perform a hash calculation. The UE’s IP Address is an input to the hash calculation.  Some additional inputs to the hash calculation are values that are known to the UE that hosts the EEC and the BSF, whch is not easily known when combined, or determined, by functions other than the UE that hosts the EEC (e.g., the SUPI of the UE). When the BSF is requested to provide information about the UE (e.g. the SUPI of the UE), the BSF may compare the result of the BSF’s calculation and the hash value that was provided by the EEC of the UE. If the result of the BSF’s calculation and the hash value that was provided by the EEC of the UE are the same, then the BSF can support the request. If the result of the BSF’s calculation and the hash value that was provided by the EEC of the UE are not the same, then the BSF does not authorize the request.

Figure 6.X.2.1, IP Address Verification

- When the EEC is triggered by the AC to get the UE ID, the EEC invokes the Eees_UE_Identifier API of the EES. The request from the EEC includes the IP address and a hash value.  The hash value is calculated based on a credential or value that is known to both the EEC and BSF.  Example values that can be used as input ot the hash calculation include the UE’s SUPI, public IP Address, private IP Address and port number, DNN of the PDU Session, and S-NSSAI of the PDU Session.

- The EES receives the request from the EEC. The EES then acts as an AF and invoke Nnef_UEId_Get request service of the NEF as described in step 2 of clause 8.6.5.3.1 of TS 23.558. The EES includes the IP Address and hash value that was provided by the EES as an input to the Nnef_UEId_Get request service invocation.

- The NEF authorizes the AF request. If the authorisation is not granted, the NEF replies to the EES with a Result value indicating authorisation failure, otherwise the NEF proceeds with the following steps. The NEF determines corresponding DNN and/or S-NSSAI information: this may have been provided by the EES or is determined by the NEF based on the requesting EES Identifier and EES Provider Information.

If the NEF has received a Port Number in step 2, based on configuration, the NEF may recognize the address received is an IP address which is different from the actual private UE IP address assigned by 5GC, i.e. the UE is behind a NAT. If so, the NEF performs steps 7 to 10. Otherwise, steps 4 to 7 are skipped.

- The NEF uses the Nnrf_NFDiscovery service operation to obtain the address of the UPF implementing NAT functionality for the UE (public) IP address. The request includes the UE (public) IP address. The NEF may also include the DNN and S-NSSAI associated with the AF ID, as well as the IP domain.

- The NRF responds with a Nnrf_NFDiscovery response message including the UPF address of the UPF implementing NAT functionality for the UE (public) IP address.

- The NEF uses the Nupf_GetPrivateUEIP_Get service operation to request UE's (private) IP address from the UPF. The request includes the UE (public) IP address and Port Number, and optionally IP domain, DNN and S-NSSAI associated with the AF ID.

- The UPF responds with the Nupf_GetPrivateUEIP_Get response message including UE's IP address and optionally, the IP domain. If the UPF has applied a NAT functionality, the UE's IP address returned by the UPF is the private UE IP address.

- When the NEF invokes the Nbsf_Management_Discovery service operation, the NEF provides the hash value to the BSF.

- The BSF will determine the SUPI that is associated with IP Address, derive the expected hash value using the same inputs as the EEC.  The calculation that is used to determine the expected hash value will be the same as the calculation that the EEC performed in step 1.

The BSF will compare the expected hash value and the hash value that was received.

- If the received hash value and the expected hash value are the same, then the BSF will respond to the NEF with a SUPI.

If the received hash value and the expected hash value are different, then the BSF will respond to the NEF with an indication that request is denied because the IP Address is not associated with the UE that made the request.

- The NEF will then use the SUPI to query the UDM and receive the UE ID from the UDM. The NEF interacts with UDM to retrieve the AF specific UE Identifier via the Nudm_SDM_Get service operation. The request message includes SUPI and at least one of Application Port ID, MTC Provider Information or AF Identifier.

- The UDM responds to the NEF with an AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the Application Port ID, MTC provider Information and/or AF Identifier.

- The NEF will send the UE ID to the EES.

- The EES sends the EES Eees_UE_Identifier response with the UE ID to the EEC.

### 6.X.3	Solution evaluation

Editor’s Note: The evaluation of this solution is FFS.

*** End of Change ***