3GPP TSG-SA3 Meeting #117	S3-242828

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Solution for secure retrieval of 5G UE Id and privacy related information

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve this contribution for TR 33.749.

# 2	References

[1]	3GPP TR 33.749 Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3

# 3	Rationale

This contribution proposes a solution for KI#1 of TR 33.749 [1], corresponding to the secure retrieval of 5G system UE Ids and privacy related information. The solution addresses both security requirements, i.e., the verification of the user information provided by the EEC, as well as the protection of the UE privacy. The overall procedure is based on a temporary identifier provided by the 5G system.

# 4	Detailed proposal

****START of CHANGES*****

## 6.Y	Solution #Y: Secure retrieval of 5G UE Id and privacy related information

### 6.Y.1	Introduction

This solution addresses the security requirements exposed in key issue #1 related to the secure retrieval of 5G system UE Id and privacy related information, i.e., the verification of the user information provided by the EEC, and the protection of the UE privacy.

The main design principle of the solution is based on the utilization of a temporary identifier (ID) as UE identity to fetch the Application Function (AF) specific UE Id from Nnef_UEId service. The temporary ID is generated by the 5G Core (e.g., UDM), it includes a randomly generated value, and it is associated to the PDU session (eventually to the AF ID).

### 6.Y.2	Solution details

Figure 6.Y.2-1 illustrates the procedure to fetch the 5G system UE Id and privacy related information.

Figure 6.Y.2-1: Procedure to fetch the 5G system UE Id and privacy related information

1. PDU session establishment / modification request from the UE to the 5G system.

2. The 5G Core (e.g., UDM) generates (per UE) an internal temporary ID (with a ramdomly generated part) for PDU session.

NOTE: The UE could request multiple temporary IDs during the same PDU session (PDU Session modification). The UE can use this to invalidate previous IDs which might have been compromised and/or at fix intervals.

3. The 5G Core returns the internal temporary ID to the UE as part of the NAS PDU session establishment/modification response.

4. The UE (EEC) uses the allocated temporary ID inside the Edge application signaling in the communication with the application function AF (e.g., EAS).

5. The AF uses the received temporary ID to invoke the Nnef_UEId API intended to fetch the 5G system UE identifier, i.e., AF specific UE Id, as per the procedure described in TS 23.502 figure 4.15.10-1 [8].

6. The NEF verifies the temporary ID, authorizes the request coming from the AF and accordingly retrieves from the 5G Core the AF specific UE Id corresponding to the temporary ID.

7. The NEF replies to the AF with the corresponding AF specific UE Id.

8-9 (Optional). Assuming the application logic requires to fetch the UE location, a location request is sent to the NEF using the AF specific UE Id, and the corresponding information is provided by the 5G system through the NEF.

10. The AF responds to the UE (EEC).

### 6.Y.3	Evaluation

The solution requires the generation of a new temporary ID identifier by the 5G Core, that needs to be used in the invocation of Nnef_UEId API services.

The solution avoids the mapping of potentially long-term and easily inferred identifiers, with the 5GS UE Id, to block malicious applications from performing UE tracking type of attacks.

Editor’s Note: Further evaluation is to be defined.

****END of CHANGES*****