SA WG3 Meeting S3#117	S3-242875

Maastricht, Netherlands 19 - 23 August 2024                    revision of S3-24wxyz

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for KI#1 on Authorization of spatial anchor services with multiple VAL-S

Document for:	Approval

Agenda Item:	5.18

Work Item / Release:	FS_Metaverse_Sec/Rel19

# 1	Decision/action requested

Approve the solution added to TR 33.721

# 2	References

[1]	3GPP TR 23.700-21 Study on Application enablement architecture for mobile metaverse services

[2]	3GPP TR 33.721 Study on security aspects of 5G Mobile Metaverse services

[3]	3GPP TS 23.434 Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows

[4]	3GPP TS 33.434 Security aspects of Service Enabler Architecture Layer for Verticals (SEAL)

[5]	3GPP TS 33.122 Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs

# 3	Rationale

In Solution #2 of TR 23-700-21, Support for spatial anchor management of 23-700-2, a VAL server may include following information in the request when create a spatial anchor:

- service information of the product to associate it with the spatial anchor,

- access control rules defining which entities are permitted to discover and access the spatial anchor,

- customer premise information (e.g. a residence, office, or shop).

- spatial anchor discoverable visibility levels like universal to facilitate shared spatial anchor discovery

When one VAL server discovers or manages the spatial anchor created by another VAL server, the access control policies from the creating VAL server should be considered to authorize the spatial anchor services to the accessing VAL server.

The contribution proposes a solution for KI#1 to authorize one VAL server/SEAL client to access spatial anchor created by another VAL server/SEAL client.

# 4	Detailed proposals

All content is new.

# * * * * First change* * * *

## 6.X	Solution #X: Solution for KI#1 on Authorization supporting spatial localization service with CAPIF Core Function (CCF)

### 6.X.1	Introduction

The solution addresses KI#1 on Authorization supporting spatial localization service.

In Solution #2 of TR 23-700-21, Support for spatial anchor management of 23-700-2, a VAL server may include following information in the request when create a spatial anchor:

- service information of the product to associate it with the spatial anchor,

- access control rules defining which entities are permitted to discover and access the spatial anchor,

- customer premise information (e.g. a residence, office, or shop).

- spatial anchor discoverable visibility levels like universal to facilitate shared spatial anchor discovery

When one VAL server discovers or manages a spatial anchor created by another VAL server, the access control policies from the creating VAL server should be considered to authorize the spatial anchor services to the accessing VAL server.

The solution proposes to authorize one VAL server/SEAL client to access spatial anchor created by another VAL server/SEAL client with considering the access control polices from the creating VAL server.

### 6.X.2	Solution details

A VAL server (VAL server1) creates a spatial anchor in a SEAL server which including access control rules/authorization policies in the creation request and the SEAL server synchronizes authorization policies with CAPIF Core Function (CCF) together with the VAL server1 information. When another VAL server (VAL server 2) requests access token from CCF for the spatial anchor, CCF checks the authorization policy of the spatial anchor. If VAL server2 is allowed to access the spatial anchor, CCF generates an access token and provides it to VAL server2. The VAL server2 initiates a spatial anchor service request along with the access token to the SEAL server. The SEAL server, upon successful validation of access token, provides the spatial anchor service to the VAL server2.

#### 6.X.2.1	Procedure of authorization of spatial anchor service with multiple VAL server

Figure 6.x.2.1-1 Procedure of authorization of spatial anchor service with multiple VAL server

0.  VAL server1 and server2 are onboarded to CCF and authenticated with CCF. VAL server2 is authorized by CCF and access token for creation of spatial anchor has been obtained from CCF.

1. VAL server1 sends a spatial anchor creation request to SEAL server with the spatial anchor information and access token obtained from CCF.

2. SEAL server validates the access token against the service request and creates a spatial anchor, e.g. with spatial anchor id set to “anchor_123”.

3. SEAL server sends spatial anchor creation response to VAL server1.

4. SEAL server publishes the spatial anchor information, e.g. spatial anchor id (anchor_123), owner of the spatial anchor (VAL server 1) and the authorization policies associated with the spatial anchor to CCF.

5. CCF stores the spatial anchor information in the VAL server1 profile.

6. VAL server 2 sends access token request to CCF to access the spatial anchor (anchor_123).

7. CCF authorizes the request based on local policies and authorization policies of the spatial anchor (anchor_123) presented in VAL server 1 profile. If VAL server2 is authorized, generates an access token.

NOTE: Optional CCF may implicitly ask authorization from VAL server1 based preconfigured policies.

8. CCF sends the access token to VAL server2.

9. VAL server2 sends request to SEAL server to access the spatial anchor, along with the access token received from CCF.

10. SEAL server validates the access token and service request and perform the request on successful validation.

11 SEAL server sends the spatial anchor response to VAL server2.

### 6.X.3	Evaluation

TBD

# * * * * End of changes * * * *