3GPP TSG-SA3 Meeting #117	S3-242878

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	key issue for no transmit zones

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR proposal for TR 33.759

# 2	References

[1]	3GPP TR 23.700-59

# 3	Rationale

This contribution proposes a new key issue for no transmit zones based on progress in SA2 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## X.1	Key issue # X: supporting NTZ and multiple USS

### X.1.1	Key issue details

In the TR 23.700-59 [2], the following restriction to UE is agreed as one of in the interim conclusions to support the No Transmit Zones (NTZ):

- No transmissions in the restricted frequency bands are allowed from a UAV UE located in the NTZ (in three dimensions to consider height restrictions), regardless of the service type.

In addition, NTZ assistance information can be made available to the UAV UE in multiple ways, including

- provided/updated by the AF (e.g. NTZ AF, USS, TPAE)

In this key issue, the scenario that NTZ assistance information is provided by the AF/USS to the 3GPP network is considered. The network is assumed to interface with multiple USS. This assumption is in line with the scope of the TR 23.700-59 [2]. The objective of this key issue is to study potential security issues arising from providing NTZ information to the network from multiple USSs.

The 3GPP network interfaces with mulitple USS through NEF. Each USS may serve different set of UAVs. In the case that one USS, e.g., USS A, sends the NTZ assistance information to the 3GPP network, it is not clear whether this NTZ information is applicable to only UAVs served by this USS (i.e., USS A) or it is applicable to all UAVs within a geography area (as indicated in NTZ assistance information). It is notable that NTZ assistance information itself does not include USS information. This ambiguity may cause potential security threats to the 3GPP system or the UAS system.

### X.1.2	Threats

In case fake NTZ information is sent from one compromised or one mischievious third-party USS (e.g., USS A), all UAVs, irrespective of whether they are served by this USS (i.e., USS A), will be restricted. In other words, all UAVs served by the whole network will be denied of services once one USS is compromised.

### X.1.3	Potential security requirements

The 5G system shall provide mechanisms to mitigate the potential risks of DoS to UAVs due to faked NTZ information from a compromised third-party USS.

***  END OF CHANGES  ***