3GPP TSG-SA3 Meeting #117	S3-242883

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#4 evaluation

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR proposal for TR 33.759

# 2	References

[1]	3GPP TR 33.759

# 3	Rationale

This contribution proposes text to evaluate Sol#4 in TR 33.759 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.4	Solution # 4: revocation supporting multiple USS

### 6.4.1	Introduction

This solution addresses the key issue #1. The solution adapts the revocation procedure in TS 33.256 [4] to support multiple USS.

### 6.4.2	Solution details

With reference to the clause 5.2.1.5 in TS 33.256 [4], the steps are amended as follows to support multiple USS:

1-2. no change to the corresponding steps in the clauses 5.2.1.4 in TS 33.256 [4].

3a or 3b. The UUAA revocation message that the UAS NF sent to the target AMF or SMF also includes the USS identifier.

NOTE 1:	As stated in 5.2.1.6 in TS 33.256 [4], the USS identifier is based on the security link on the interface between UAS NF and USS (e.g., the identity mapped during link establishment or the identity in certificate).

3c-4. no change.

4a or 4b. The UUAA revocation indication message that the target AMF or the SMF sent to the UE contains also the USS identifier.

5. The UE on receiving the revocation indication only delete authorization data corresponding to the USS identifier.

### 6.4.3	Evaluation

This solution addresses the key issue #1. It amends the revocation procedures in the TS 33.256 [4] in order to support multiple USSs. It has the following impacts:

- UAS NF: sends the USS identifier to the UAV since the UAV needs to delete authorization data specific to the USS, not all authorization data.

This solution is in line with the conclusions in TR 23.700-59 [2].

***  END OF CHANGES  ***