3GPP TSG-SA3 Meeting #117	S3-242885

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#5 evaluation

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-32

# 2	References

[1]	3GPP TR 33.700-32

# 3	Rationale

This contribution proposes evaluation text for Sol#5 in TR 33.700-32 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.5	Solution #5: User authentication and authorization

### 6.5.1	Introduction

This solution addresses the key issue #1.

The solution reuses the NSSAA procedure TS 33.501 [3] and adapts it to authenticate and authorize a human user using the UE. It allows restriction to user through authorization outcome.

### 6.5.1	Solution details

Assuming the human user with a user identifier (UID) is using a UE with a 3GPP subscription identified by the UE’s SUPI to access services via the 5GS. Before authentication of the UID, the Primary authentication for the UE needs to be performed. It is also assumed that the networks slice identified by S-NSSAI is supposed to provide the service for the UE and the user.

With reference to the figure 16.2-1 in TS 33.501 [3], the user identity authentication and authorization procedure is described as follows:

1. UE sends a Registration Request.

NOTE: How the user triggers the UE to send the registration request is out of scope.

2. For an initial Registration Request, the AMF shall invoke Primary authentication as described in TS 33.501 [3]. For a subsequent Registration Request, the Primary authentication may be skipped if the UE has already been authenticated and the AMF has valid security context.

3. The AMF shall determine whether user identifier authentication and authorization (UIAA) procedure is required, based on information stored locally or from UDM. If yes, the AMF triggers the UIAA procedure. The UIAA procedure can be adapted from NSSAA procedure as shown in 16.3 in TS 33.501 [3] with modification as follows:

NOTE: the step “3-x” is corresponding to the step “x” in clause 16.3 in TS 33.501 [3]

3-1: The AMF may trigger the start of the UIAA procedure.

Editor’s note: How the AMF knows to perform user authentication is ffs.

3-2: The AMF may request the EAP ID for EAP authentication.

3-3: The UE provides the EAP ID to the AMF. The EAP ID is included in the EAP message which is transparent to the AMF. The UE may provide UID to the AMF as well.

NOTE: UID may be the same or different from EAP ID. The UID format is out of scope of this solution.

Editor’s Note: EAP ID is used for authentication with the AAA-S. If UID is different from EAP ID, then how user authentication based on UID is performed is FFS.

3-4: The AMF sends the GPSI, EAP ID and the UID if available to the NSSAAF.

3-5: 5.	The NSSAAF forwards the message to the AAA-S directly or through AAA-P if available.

3-6 to 3-11: EAP-messages are exchanged with the UE. One or more than one iterations of these steps may occur.

3-12.	EAP authentication completes. The EAP-Success/Failure message is delivered to the NSSAAF/AAA-P along with GPSI and UID. The message may include any restriction information imposed to the UID, e.g. tiers of services/QoS, service duration etc.

Editor’s Note: Whether the AAA-S can provide the restriction information, e.g. tiers of services/QoS, service duration, is FFS.

3-13.	The NSSAAF sends the UIAA result (Success/Failure), GPSI, UID to the AMF.

3-14.	The AMF transmits the UIAA result to the UE.

3-15. Based on the UIAA result, the AMF initiates the UE Configuration Update procedure.

Editor’s note: Whether the AMF initiates UE configuration update and what to be updated should be aligned to SA2

### 6.5.3	Evaluation

This solution addresses the key issue #1.

This solution reuses the NSSAA procedure and adapts it to authenticate and authorize a user identifier with few changes highlighted as follows:

UE: includes the user identifier in the registration request.

AMF: triggers the user identifier authentication procedure, e.g., based on subscription information at UDM

AAA: exchanges messages with the UE assisted by the AMF to authenticate and authorize the user identifier.

***  END OF CHANGES  ***