3GPP TSG-SA3 Meeting #117	S3-242891

Maastricht, Netherlands 19 - 23 August 2024

Title:	Reply-LS on User Identities and Authentication Architecture

Response to:	S3-242724 LS on User Identities and Authentication Architecture from SA2

Release:	Release 19

Work Item:	FS_UIA_ARC

Source:	3GPP SA3

To:	SA2

Cc:	SA3-LI, SA1

Contact person:	lei.zhongding@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 would like to thank SA2 for the LS on User Identities and Authentication Architecture. SA3 has discussed the questions raised and provided the answers to Questions 1a, 1b and 2 as follows:

- Q1a: During the discussion related to Key Issue #1 and Key Issue #2 most companies believe that it is appropriate to send the user identifier in NAS (e.g., to trigger authentication by the network and to activate the user with a subscription). Different from the majority view, some companies objected to this conclusion and raised the issue of trust of the user identifier and credentials for the MNO (HPLMN) and questioning whether the HPLMN will be able to trust the user identifier and credentials.

While considering that the views from majority of companies being,

- the user identity and credentials that will be used to identify the user in HPLMN might not be stored in UICC. For example, a User Interface could be used to retrieve user identifier and credentials directly from the human user; or the User Interface could identify the human (e.g., based on biometric authentication) and related human user identifier and credentials could be retrieved by the operating system (e.g., from the UICC or other local storage).

- procedures and methods similar to NSSAA will be used to authenticate the user.

SA2 requests SA3 to provide feedback if their views are inline to the majority view as described above.

[SA3]: SA3 confirms that it is appropriate to send the user identifier in NAS to trigger authentication by the network and to activate the user with a subscription. There is no security issue to send the user identifier based on SA3 study. For the potential privacy issue of sending a user identifier, it will be addressed in the SA3 study and concluded in due time.

As far as the storage of the user credentials is concerned, no security requirements have been identified in SA3. The storage entity can be either UICC or ME. This depends on deployment or agreement between the operator and the third parties.

SA3 confirms that the procedures and methods similar to NSSAA and secondary authentication are appropriate to be used to authenticate the user.

- Q1b: Regarding the Key Issue #3 of TR 23.700-32, can any existing information of the UE subscription with whom the user identifier is active, or new information related to the user to be allowed to be exposed to an authorized AF, and if so, is user consent required for the exposure of this information? Specifically, we would like to understand the user consent requirements for the following new information:

- the verification result indicating whether the User Identifier authentication is successful.

[SA3]: Firstly, whether information related to the user can be exposed to an AF depends on whether information belongs to the operator network or the AF. If information belongs to the AF, yes, it can be exposed to the authorized AF. However, if information belongs to the operator network, information, if sensitive, should be privacy protected before being exposed to an authorized AF. For example, a UE identifier SUPI can be mapped to a GPSI before exposing to an authorized AF.

Regarding the verification result indicating whether authentication is successful, it is not sensitive information and can be exposed to an authorized AF. As to the question about “user consent”, it should be noted that the “user consent” should not be mixed with AF authorization for user information. The “user consent” is a separate issue and out of scope of this study. The “user consent” refers to the framework specified in the Annex V in TS33.501. It is meant to manage subscriber permissions at the operator domain such that Network Functions of the 3GPP system can request or can be provisioned/configured with subscriber-related data in accordance with local laws and regulations.

- Q2: Regarding Key Issue #1 of TR 23.700-32, SA2 kindly requests SA3 to provide their opinion whether there is a privacy issue if the network keeps providing SMS and IMS services to a UE when a user identifier is active with the UE subscription.

[SA3]: No privacy issue on this aspect has been identified so far in SA3. It can be addressed by the SA3 study in case such an issue identified.

# 2	Actions

To SA2

ACTION: 	SA3 kindly asks SA2 to take above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)