3GPP TSG-SA3 Meeting #117	S3-242956

Maastricht,NL 19 - 23 August 2024

# Source:	Intel

# Title:	Discussion Paper FSAT Solutions

# Document for:	Discussion

Agenda Item:	5.7

# 1	Decision/action requested

It is requested that to endorse the Conclusion and Proposal

# 2	References

[1]	 TR 33.700-29:	" Study on Security Aspects of 5G Satellite Access

in the 5G architecture "

# 3	Rationale

The document compares all the solutions in FSAT TR 33.700-29[1] and proposes the architecture principles that can be adopted based on the SA2 conclusions for store and forward.

# 4	Detailed proposal

This paper aims to discuss and evaluate the proposed security solutions for Store and Forward (S&F) Satellite Operations in 3GPP networks, addressing the key issues identified by SA3. Key Issue #1: Security Protection in Store and Forward Satellite Operations has following security challenges described:

Incomplete authentication due to intermittent connectivity.
Potential for Man-in-the-Middle (MITM) or impersonation attacks.
Risks of denial of service (DoS) due to resource exhaustion.

The following solutions have been analyzed to address the security challenges in S&F operations. Each solution is grouped based on the architectural approach.

Comparative Analysis

The following updates should be made to the comparative analysis:

4.1 Dependency on SA2 or RAN

- Yes: Solutions 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 22, 23, 24, 25

  - All solutions involve some dependency on SA2 or RAN for implementation, coordination, or procedural adjustments.

4.2 Key Issues Addressed

- Key Issue #1: Security Protection in Store and Forward Satellite Operations:

  - Solutions 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 22, 23, 24, 25

  - Each solution addresses various aspects of security protection, focusing on authentication, confidentiality, integrity, and DoS prevention.

4.3 Architecture Option

- Split MME Architecture:

  - Solutions 1, 5, 9

  - These solutions involve a split architecture, with key components onboard the satellite and others on the ground.

- Full CN Onboard:

  - Solutions 2, 4, 5, 6, 7, 8, 10, 11, 12, 13, 14, 15, 16, 17, 18, 22, 23, 24, 25

  - These solutions assume the entire core network is onboard the satellite, enabling continuous operations even with intermittent connectivity.

4.4 Re-use of Legacy Procedures

- Partial Re-use: Solution 1

  - Adapts the AKA procedure for Split MME architecture.

- Solution 9: Utilizes standard authentication vectors from HSS, adapted for split MME functionality.

- Adapted Legacy Procedures: Solutions 2, 4, 6, 11, 12, 14, 15, 16, 17, 18, 22, 23, 24, 25

  - Involves adapted procedures for provisional registration, key management, NAS adjustments, UE context management, subscriber key separation, OTP-based authorization, pre-authentication checks, MO data transmission authorization, back-off timer management, integrity protection activation, selective SAT group management, secure NAS message delivery during S&F operations with onboard RAN, expedited authentication in 5GS, and DoS prevention before security context establishment.

- Full Use of Standard TS 33.401: Solutions 5, 10, 13

  - Applies standard procedures for onboard UDM and secure token-based attach/registration, with adaptations for onboard HSS.

- Inverse AKA Procedure: Solution 7

  - Focuses on optimizing the subsequent authentication using pre-provisioned AVs.

- ECCSI-Based: Solutions 4, 8

  - Uses ECCSI for mutual authentication before the primary AKA.

4.5 Normative Work Required

- Inverse AKA Specification: Solution 1

  - New specifications for Inverse AKA.

- Key Management: Solutions 2, 11, 12, 13, 18, 22, 24, 25

  - Requires normative work on derived key management, handling UE context in tokens, subscriber key separation, synchronization of onboard HSS with ground HSS, secure key management within selective SAT groups, expedited authentication procedures in 5GS, and early authentication using KSAT.UE to prevent DoS attacks.

- Provisional Registration Specification: Solution 4

  - Specifications needed for bundling authentication and provisional registration.

- UDM Synchronization: Solution 5

  - Needs guidelines on UDM sync between satellite and ground networks.

- NAS Timers Adjustment: Solution 6

  - Normative work on adjusting NAS timers for S&F.

- AV Provisioning: Solution 7

  - Requires specifications for provisioning AVs for subsequent authentication.

- ECCSI Implementation: Solution 8

  - Normative work required to implement ECCSI for mutual authentication.

- Token-Based Attach/Registration: Solutions 10, 11

  - Normative work required to define the structure, protection, and transmission of UE authentication tokens.

- OTP Authorization: Solution 14

  - Normative work required to define the mechanism for OTP generation, distribution, and verification in uplink NAS message authorization.

- Authorization Before Authentication: Solution 15

  - Requires specifications for integrating UE authorization checks before the authentication process during the attach procedure under S&F operations.

- Authorization During MO Transmission: Solution 16

  - Normative work required to specify the procedures for authorizing UEs during MO data transmission in S&F operations, including the handling of authorization information and reject messages.

- Back-off Timer Implementation: Solution 17

  - Normative work required to specify how the back-off timer is calculated based on orbit information and how it integrates with the attach procedure to avoid S&F operation issues.

- Integrity Protection Activation: Solution 18

  - Requires normative work to specify the conditions under which UP integrity protection should be activated during S&F operations, as well as guidelines for handling data storage and prioritization based on integrity verification.

- Selective SAT Group Establishment: Solution 22

  - Requires normative work to define the establishment, management, and security of selective SAT groups, including key sharing and paging procedures over ISL.

- Secure NAS Message Delivery: Solution 23

  - Normative work required to specify the mechanism for generating and verifying UE Auth_code and RAN_Auth_code to protect the RRC connection procedure and ensure secure NAS message delivery during S&F operations.

- Expedited Authentication Procedures: Solution 24

  - Normative work required to define the expedited authentication procedures, including the pre-fetching of authentication information and partial registration acceptance, to optimize security context establishment in 5GS during S&F operations.

- Pre-AKA Authentication Using KSAT.UE: Solution 25

  - Requires normative work to define the procedures for deriving and using KSAT.UE for early authentication between the UE and satellite to prevent DoS attacks and ensure secure initial communication in S&F operations.

- Solution 9: Reuse AKA procedure, Specification of the split MME architecture and the secure handling of IMSI and authentication vectors.

4.6 Advantages

- Enhanced Security: Solutions 1, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 22, 23, 24, 25

  - Provides increased security during intermittent connectivity, prevents DoS attacks, ensures secure key management, supports single satellite deployment with onboard HSS, adds OTP-based uplink NAS message authorization, introduces pre-authentication authorization checks, ensures authorized MO data transmission, prevents incomplete AKA procedures during S&F operations, enhances integrity protection for stored data, introduces secure AS security context establishment with selective SAT groups, prevents unauthorized NAS message delivery with onboard RAN, optimizes authentication procedures in 5GS, and prevents DoS attacks before security context establishment..

- Key Protection: Solutions 2, 12, 13, 18, 22, 24, 25

  - Ensures security using derived keys, reducing the risk of key compromise, with additional protection through subscriber key separation, enhanced integrity protection for stored data, secure key management within selective SAT groups, expedited authentication in 5GS, and early authentication using KSAT.UE.

- Bundled Authentication: Solution 4

  - Streamlines the process, securing small data transmissions.

- Continuous Authentication: Solutions 5, 9, 13, 24

  - Maintains continuous authentication even during feeder link unavailability, supported by onboard HSS and expedited authentication procedures.

- Re-use of Legacy Procedures: Solutions 6, 9, 12, 13, 18, 22, 23, 24, 25

  - Leverages existing NAS procedures, EPS AKA, and integrity protection mechanisms with minimal changes, while introducing secure NAS message delivery during S&F operations with onboard RAN, expedited authentication in 5GS, and early authentication using KSAT.UE.

- Optimized Subsequent Authentication: Solution 7

  - Simplifies subsequent authentications using pre-stored AVs.

- Seamless Handover: Solution 11

  - Ensures secure and seamless handovers between satellites by maintaining UE context continuity.

- Uplink NAS Authorization: Solution 14

  - Prevents unauthorized uplink NAS messages, mitigating the risk of storage space exhaustion due to malicious activity.

- Pre-authentication Authorization: Solution 15

  - Prevents resource wastage by ensuring only authorized UEs proceed to authentication, enhancing overall security.

- Integrity Protection: Solution 18

  - Ensures only verified data is stored during S&F operations, preventing malicious data from being stored and processed.

- Selective SAT Group Management: Solution 22

  - Optimizes paging and UE context management across multiple satellites, ensuring reliable communication and reduced latency in UE data transfers.

4.7 Disadvantages

- Complexity: Solutions 1, 6, 11, 12, 13, 14, 15, 16, 17, 18, 22, 23, 24, 25

  - Increased complexity due to procedural adaptations, UE context management, subscriber key separation, token management, OTP-based authorization, pre-authentication checks, MO data transmission authorization, back-off timer management, integrity protection activation, selective SAT group management, secure NAS message delivery during S&F operations with onboard RAN, expedited authentication procedures in 5GS, and early authentication using KSAT.UE.

- Key Management Challenges: Solutions 2, 11, 12, 13, 18, 22, 24, 25

  - Complexity in managing derived keys across the network, maintaining UE context in tokens, synchronizing onboard HSS with ground HSS, securely managing keys within selective SAT groups, ensuring the pre-fetching of authentication information in 5GS for expedited procedures, and managing KSAT.UE for early authentication.

- Token Management: Solutions 4, 10, 11, 14

  - Requires careful management of provisional registration tokens, UE authentication tokens, and OTPs.

- Risk of Satellite Compromise: Solutions 5, 12, 13, 18

  - Vulnerability if onboard HSS or subscriber keys are compromised, especially if the keys are stored across multiple satellites or used without integrity protection.

- AV Management: Solution 7

  - Managing and storing AVs adds to the complexity.

- ECCSI Resource Usage: Solution 8

  - Increased computational resources required for ECCSI.

- Pre-authentication Authorization Checks: Solutions 15, 16

  - Complexity in managing authorization data during MO data transmission and prior to the authentication process, which may require additional procedural steps and resource allocation.

- Back-off Timer Implementation: Solution 17

  - Requires precise orbit information and complex timer management to ensure accurate calculation and synchronization with the attach procedure.

- Integrity Protection Activation: Solution 18

  - Complexity in determining when to activate integrity protection and managing data storage based on integrity verification.

- Selective SAT Group Management: Solution 22

  - Complexity in establishing, managing, and securing selective SAT groups, including key sharing and paging procedures over ISL.

- Secure NAS Message Delivery: Solution 23

  - Additional steps are required to generate and verify UE Auth_code and RAN_Auth_code, adding to the complexity of NAS message delivery during S&F operations.

- Expedited Authentication Procedures: Solution 24

  - Increased complexity due to the need for pre-fetching authentication information, coordinating between onboard and ground-based AMF, and managing partial registration acceptance, with potential impacts on existing 5G-AKA procedures.

- Pre-AKA Authentication Using KSAT.UE: Solution 25

  - Adds complexity in managing the early authentication process, including the derivation and verification of KSAT.UE, with additional computational resource requirements for generating and verifying MACs.

# 5	Conclusions and Proposals

#### The following security principles are proposed for the normative phase:

- Any proposed solutions for S&F operations should strive to reuse and adapt existing 3GPP procedures, such as the AKA mechanisms.

- Changes to the core 3GPP procedures (such as AKA, KDF, and NAS/AS security) should be minimized. The focus should be extending and optimizing existing processes rather than introducing entirely new frameworks.

- Given the intermittent connectivity inherent in S&F operations, security mechanisms should be designed to maintain robust authentication, confidentiality, integrity, and anti-replay protections, even under the constraints of limited connectivity.

- The solution should include proactive measures to mitigate the risk of DoS attacks.

- The baseline for security architecture in S&F operations should align with SA2’s conclusion on the split MME architecture. This architecture, which involves a split between MME functionality onboard the satellite (MME-SAT) and on the ground (MME-GND), should be the reference model for implementing security measures.

- Proposed solutions should have minimal normative impact, focusing on clarifying and enhancing existing specifications rather than introducing entirely new requirements.