3GPP TSG-SA3 Meeting #117	S3-242983

# Maastricht, Netherlands 19 - 23 Aug 2024

# Source:	OPPO

# Title:	Update KI on protection of information during AIoT service communication

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

It is proposed to update KI on Protection of information during AIoT service communication.

2	References

N/A

3	Rationale

The contribution proposes to update the KI #4 on protection of information during AIoT service communication. Specifically, this contribution analyzes the security threats and security requirements of communication for the Inventory only case.

4	Detailed proposal

**** Start of Change****

5.4	Key issue #4: Protection of information during AIoT service communication

5.4.1	Key issue details

As per TS 22.369 [2], Ambient power-enabled IoT (AIoT) services aim to support various use cases, including inventory taking, sensor data collection, asset tracking, and actuator control. These services intended to operate with lower power consumption and complexity than the existing IoT technologies such as eMTC, NB-IoT, and RedCap. To fulfil these requirements, AIoT devices require a communication capability.

TR 23.700-13 [4] has a key issue pertains to support the AIoT services, i.e., Inventory and Command. The inventory operation is to discover the AIoT device(s) in a specific area, e.g., to read the AIoT device ID.

According to the agreements in RAN2, as baseline, the “inventory only” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS.

The A-IoT paging message may include AIoT device filter information (e.g., partial AIoT device ID). The AIoT device responds to the Inventory with the AIoT device ID.

From a security perspective, security mechanisms to protect the information transmitted during AIoT service communication need to be supported. Failure to provide such security mechanisms will lead to various attacks such as eavesdropping, manipulation and/or unauthorized transmission of the information during AIoT service communication.

5.4.2	Security threats

Failure to protect the integrity of Inventory response message will lead to attacks such as fabrication, modification, or removal of the AIoT ID resulting in the failure of Inventory.

Failure to protect the confidentiality of Inventory response message may lead to attacks such as eavesdropping attacks of the AIoT ID.

NOTE: But the AIoT ID may not be related to privacy.

5.4.3	Potential security requirements

The 5GS shall provide a means to support integrity protection of the Inventory response message.

The 5GS may provide a means to support confidentiality protection of the Inventory response message.

**** End of Change****