3GPP TSG-SA3 Meeting #117	S3-243005

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Nokia

# Title:	Solution 22 updates with evaluation

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the solution 22 evaluation updates added to TR 33.700-29

# 2	References

[1]	3GPP TR 33.700-29 Study on Security Aspects of 5G Satellite Access in the 5G Architecture phase 3

[2]          3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture

# 3	Rationale

The contribution proposes an update to existing solution#22 for AS security context establishment with store and forward operations for Editor note removal. Detailed evaluation is provided in the section 6.22.3 of this document.

Editor’s Note: Whether this solution is aligned with SA2 is FFS.

Editor's Note: Further evaluation is FFS.

# 4	Detailed proposals

Updated content in the change part is new.

# * * * * First change* * * *

## 6.22	Solution #22: AS security context establishment with store-and-forward operations

### 6.22.1	Introduction

This solution proposes the following:

- Establish a (selective) SAT Group/constellation. Every Satellite which is maintaining a Service link to a UE/IoT acts as the primary-SAT and is establishing a communication to other neighbour Satellites. The primary-SAT establishes Satellite Group/constellation which is used to transfer UE/IoT contexts and/or Data. In case the UE/IoT and/or Satellite is moving, then the primary-SAT ‘role’ also moves to the new primary-Satellite. AMF can select other satellites to page the UEs. And other satellites forward the paging request to the right satellite too.

- Alternatively, or additionally, selective SAT Group information is made aware at the AMF. So that AMF can select single alternative satellite to page the UE.

### 6.22.2	Solution details

Figure 6.22.2-1: Introduction of selective SAT group

The Application Functions (AF) for some reasons might need to connect to a specific UE (IoT), because the AF wants to share data for instance. For this purpose, the UE (IoT) needs to be paged. The UE (IoT) paging procedure will need to be triggered by the AMF.

The AMF is connected to different satellites (SAT#1, SAT#2, …) and the UE (IoT) is connected to SAT#3 (for instance). Now if we assume the AMF is sending paging information for UE (IoT), BUT the Feeder link to the SAT#3 is not available (=active). In this case the AMF cannot reach out to the UE (IoT) on a direct path.

The UE (IoT) is connected to the SAT#3, but the SAT#3 has no Feeder link active.

The SAT#3 is acting as Primary SAT (pSAT) and is establishing a Selective SAT Group. This selective SAT group (SSG) consists of at least two satellites, i.e., one Primary SAT and another one Secondary SAT. New satellites can be added to the Selective SAT group or existing can be removed. The pSAT is maintaining an inter-SAT communication to every secondary SAT (star-topology). This star-topology is a logic inter-SAT deployment, i.e., every SAT that is maintaining an active Service link to a UE (IoT) is acting as a primary SAT.

This can be seen as a logical meshed deployment.

(1) The UE (IoT) is connected via a service link with SAT#3. The SAT#3 is acting as the primary SAT for this one UE (IoT). The SAT#3 is owning the KgNB and is generating all relevant keys according to the key-hierarchy defined by TS 33.501.

(2) The primary SAT#3 is establishing a Selective SAT Group and is setting up connections to all satellites which are belonging to this Selective SAT Group.

(3) The SAT#3 does not have any longer an active Feeder link.

(3a) The SAT#3 is sending notifications to all SAT Group members and is informing about the missing/inactive (own) Feeder link. The SAT#3 is managing any keys that need to be shared with SAT group members in case of UE handover. This information exchange is via RRC.

(4) The AMF is triggering paging procedure for the UE (IoT) via all the active Feeder link(s). In this deployment scenario the AMF is sending paging messages towards SAT#1 and SAT#2.

(5) The SAT#1 and SAT#2, both Satellites, receive independent the paging request for UE (IoT), both are paging the UE (IoT) within their own ‘tracking area’.

(5a) The SAT#1 and SAT#2, both Satellites send a Paging Request towards the SAT#3. Due to this, the SAT#3 is receiving both paging requests and is triggering paging for that UE (IoT) device in own ‘tracking area’. This paging message is via XnAP.

(6) The UE (IoT) is receiving the paging request and is responding with a Random-Access Procedure.

Figure 6.22.2-2:

(1) The UE (IoT) is connected via a service link with SAT#3. The SAT#3 is acting as the primary SAT for this one UE (IoT). The SAT#3 is owning the KgNB and is generating all relevant keys according to the key-hierarchy defined by TS 33.501.

(2)The primary SAT#3 is establishing a Selective SAT Group and is setting up connections to all satellites which are belonging to this Selective SAT Group. The SAT#3 informs AMF about the Selective SAT Group. i.e., for SAT#3, SAT#1 and SAT#2 are backup. AMF stores this information. AMF could also configure this SAT group (S&F monitoring list of SATs).

(3) The SAT#3 does not have any longer an active Feeder link.

(3a) The SAT#3 is sending notifications to all SAT Group members and is informing about the missing/inactive (own) Feeder link. The SAT#3 is managing any keys that need to be shared with SAT group members in case of UE handover. This information exchange is via RRC.

(4) When AMF wants to trigger the paging procedure for the UE (IoT) and finds out that SAT#3 is not accessible, then the AMF triggers paging procedure for the UE (IoT) via any one SAT available in the Selective SAT Group with active Feeder link(s). In this deployment scenario the AMF is sending paging messages towards SAT#1 (or SAT#2).

(5) The SAT#1 (or SAT#2) Satellite receive the paging request for UE (IoT), SAT#1 pages the UE (IoT) within their own ‘tracking area’.

(5a) The SAT#1 Satellites sends a Paging Request towards the SAT#3 as well. Due to this, the SAT#3 is receiving a paging request and is triggering paging for that UE (IoT) device in own ‘tracking area’. This paging message is via XnAP.

Editor’s Note: The meaning of own ‘tracking area’ in the selective SAT group/constellation and whether it is related to UE location is FFS.

(6) The UE (IoT) is receiving the paging request and is responding with a Random-Access Procedure.

Editor's Note: This solution relies on communication over ISL.

Editor's Note: How to establish and manage the SAT group is FFS.

Editor’s Note: How the solution aligns with current 3GPP standards, and what are the implications for backward compatibility is FFS.

Editor’s Note: How the solution safeguards against potential security vulnerabilities in the key sharing process over ISL is FFS.

MME architecture: Above solution will work also for MME architecture.

### 6.22.3	Evaluation

•            Assumptions: The eNB / gNB is on board of SAT and AMF/MME is in not in Satellite.

•            Dependency on SA2 or RAN: Relates to the SA2 conclusion clause 8.2 of TR 23.700-29, where monitoring list updates to UE is expected to be consider in normative work.

•            Relevant KI and Potential Security Requirements addressed: KI#1, Requirements 1.

•            Architecture option: RAN onboard the satellite

•            Re-use of legacy security procedures: Complete re-use of AKA procedure.

•            Normative work expected for this solution: In context of SA2 conclusion, clause 8.2 of TR 23.700-29, selective SAT group can be considered as monitoring list. Accordingly, the normative work in SA2 can be considered for this solution.

•            Advantages of the solution: During no-feeder link, other SATs can be used to send signalling or small data packets, which optimizes the procedure. The UE will anyhow monitor those S&F monitoring SAT list (Reference: Note and conclusions from 8.2 of TR 23.700-29), so all the messages need not be stored in one SAT for longer time instead forwarded via its SAT group to target UE.

•            Disadvantages of the solution: This solution relies on ISL communication.

•            Impacted entities: eNB/gNB, AMF, UE. No impact on legacy AKA procedures defined by SA3.

# * * * * End of changes * * * *