3GPP TSG-SA3 Meeting #117	S3-243037

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Addressing the EN on Human User authentication of through NAS procedure

# Document for:	approval

Agenda Item:	5.10

# 1	Decision/action requested

It is proposed to consider evaluation to solution 6 for the KI#1 in TR 33.700-32.

# 2	References

NA

# 3	Rationale

This contribution proposes text to address ENs for Sol#6 in TR 33.700-32.

# 4	Detailed proposal

*****Start of Change *****

## 6.6	Solution #6: Human User authentication of through NAS procedure

### 6.6.1	Introduction

This solution addresses key issue #1: "Authentication and Authorization of Human User ID". The solution focuses on the authentication procedure.

### 6.6.2	Details

The user ID is sent by the UE in the Registration Request message. When the AMF receives the Registration Request message, the AMF first perfroms the regstriation procedure for the UE. The user authentication procedure will run after the Primary authentication.

Figure X-1: User Authentication Procedure for human user

1. The UE that the user is using may or may not have registered to the 5GC.

2. A user logs onto the UE.

3. The UE sends the Registration Request message to the AMF. If the UE registered to the 5GC before, then the 5G-GUTI is included, otherwise, the SUCI is included. Additionally, the user ID will be carried in the Registration Request message.

4. The AMF continutes the Registration Procedure for the UE. If the UE registration fails, the AMF will terminate the procedure the same way as in TS 23.502[4]. If the UE registration compeltes, the UE runs the user authentication procedure.

5. After completing the UE registration procedure, the AMF starts to run user authentication procedure.

6-10. Comparing to primary authentication procedure define in TS 33.501[3], only EAP method is used for user authentication. AUSF and UDM are reused for user authentication procedure.

Note: The primary authentication and the user authentication are separate procedures. There will be no effect on the UE primary authentication method from the user authentication process. In this solution, the specific user authentication method is based on EAP-TLS.

The user authentication procedure based on EAP-TLS method applies to the cases where a credential is preconfigured in the UE and the network for user authentication. The credential associated with the user identifier is preconfigured in the UE before user activation. The credential is also preconfigured in the 5GC (UDM) with the associated user identifier.

11. The AMF sends the Registration Accept message to the UE. The Registration Accept message further contains the authentication result of the user authentication.

12. If EAP-Success is received, the UE will allow the user to use the UE. Otherwise, the UE will drop the user.

It is assumed that, the UE is pre-configured with User Id authorization information (e.g., authorized PLMNs, access type), and the User Id is linked to a UE subscription in the UDM/UDR.  The AMF checks subscription data and user profile for User Id from UDM/UDR to check whether User id is linked to the UE subscription.

### 6.6.3	Evaluation

TBD

*****The End*****