3GPP TSG-SA3 Meeting #117	S3-243038

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Evaluation to solution 6 to address KI#1

# Document for:	approval

Agenda Item:	5.10

# 1	Decision/action requested

It is proposed to consider evaluation to solution 6 for the KI#1 in TR 33.700-32.

# 2	References

NA

# 3	Rationale

Evaluation to solution 6 for Key Issue#1.

# 4	Detailed proposal

*****Start of Change *****

## 6.6	Solution #X: Human User authentication of through NAS procedure

### 6.6.1	Introduction

This solution addresses key issue #1: "Authentication and Authorization of Human User ID". The solution focuses on the authentication procedure.

### 6.6.2	Details

The user ID is sent by the UE in the Registration Request message. When the AMF receives the Registration Request message, the AMF first perfroms the regstriation procedure for the UE. The user authentication procedure will run after the Primary authentication.

Figure X-1: User Authentication Procedure for human user

1. The UE that the user is using may or may not have registered to the 5GC.

2. A user logs onto the UE.

3. The UE sends the Registration Request message to the AMF. If the UE registered to the 5GC before, then the 5G-GUTI is included, otherwise, the SUCI is included. Additionally, the user ID will be carried in the Registration Request message.

4. The AMF continutes the Registration Procedure for the UE. If the UE registration fails, the AMF will terminate the procedure the same way as in TS 23.502[4]. If the UE registration compeltes, the UE runs the user authentication procedure.

5. After completing the UE registration procedure, the AMF starts to run user authentication procedure.

6-10. Comparing to primary authentication procedure define in TS 33.501[3], only EAP method is used for user authentication. AUSF and UDM are reused for user authentication procedure.

Editor’s Note: It is ffs to clarify the EAP methond compared with primary authentication.

Editor’s Note: It is ffs to clarify the credential used for user authentication.

11. The AMF sends the Registration Accept message to the UE. The Registration Accept message further contains the authentication result of the user authentication.

12. If EAP-Success is received, the UE will allow the user to use the UE. Otherwise, the UE will drop the user.

Editor’s Note: It is ffs whether the UE needs to authorize the user based on the result of the UE authentication.

### 6.6.3	Evaluation

This solution satisfies all of the potential security requirements in KI#1.

The solution reuses the secondary authentication procedure and adapts it to authenticate and authorize of a user identifier in the human user scenario. It uses EAP framework so that various authentication methods (e.g, EAP-TLS) can be supported.

One of the primary security benefits of the proposed solution (based on EAP-TLS) is the ability to perform 5GC certificate validation. An attacker cannot impersonate a session participant to carry out a man-in-the-middle attack because he does not know that only legitimate parties have shared keys.

In summary, the proposed secondary authentication solution offers robust security by safeguarding user identification information, ensuring message integrity, and enabling two-way authentication.

*****The End*****