3GPP TSG-SA3 Meeting #117	S3-243040

Maastricht, Netherlands 19 - 23 August 2024

Title:	Reply-LS on Resource content filters

Response to:	S3-241733 LS on Resource content filters from CT4

Release:	Release 18

Work Item:	FS_RedInfExp_SBI

Source:	3GPP SA3

To:	CT4

Cc:

Contact person:	Bo Zhang

zhangbo6@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA3 would like to thank CT4 for the LS on Resource content filters. SA3 has discussed the questions raised and provided the following answers:

- Question: CT4 is studying Reducing Information Exposure over SBI and has captured solutions #1 and #6 in TR 29.857 to define a different access level(s) in the token returned from NRF to NF consumers. Since the Stage-2 of OAuth mechanism and the information provided in Access token claims is in the remits of SA3, CT4 kindly asks SA3 to inform CT4 on issues (if any), e.g. security issues, SA3 identifies concerning these solutions.

[SA3]: In the Service Based Architecture (SBA), only authorized NF-Consumers are allowed to access an NF-Producer's resources. Regarding the authorization granularity, the basic extent provided by the authorization token is at service level (i.e. the "scope" claim). A higher level of granularity for the authorization token can be defined with "additional scope" information within the token (e.g. the text as specified in Clause 13.4.1 of TS 33.501):

The basic extent provided by the authorization token is at service level (i.e. the "scope" claim includes allowed services per NF type). Depending on the NF Service Producer configuration, higher level of granularity for the authorization token can be defined adding "additional scope" information within the token e.g. to authorize specific service operations and/or resources/data sets within service operations per NF Service Consumer type.

NOTE 1: The additional scope(s) included within the access token add additional security checks at the NF Service Producer that authorizes the services operations, resources and NF Service Consumer type related to the additional scope(s).

...

The claims in the token shall include the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service Consumer (subject), NF type of the NF Service Producer (audience), expected service name(s) (scope), expiration time (expiration) and optionally "additional scope" information (allowed resources and allowed actions (service operations) on the resources). The claims may include a list of S-NSSAIs or NSI IDs for the expected NF Service Producer instances. The claims may include the NF Set ID and/or NF Service Set Id of the expected NF Service Producer instances.

NOTE: If the claims do not include a list of NSSAIs or NSI IDs for the target NF type, it implies the token can be used to access expected NF services of all expected NF Service Producers of the NF type based on local configuration and operator policy.

The additional scope can be defined by the operators taking into consideration the services provided by the operators.

The issues mentioned in the TR 29.857 were initiated from GSMA on the following two LSes.

GSMA LS C4-225023: "Research highlighting potential need for granular level checks using "Additional scope" under the OAuth2.0 Token Access" (https://www.3gpp.org/ftp/tsg_ct/WG4_protocollars_ex-CN4/TSGCT4_113_Toulouse/Docs/C4-225023.zip).

SA3 Replied in S3-211581 (SA3 #110) that

“3GPP provides the flexibility to operators to allow deploying additional scopes for specific NF services, and this can be achieved by defining additional scopes in 5GC APIs when appropriate. But whether OAuth2.0 is used in the network, and if yes, which scopes (service-level scope and/or additional scopes) are used, this should be up to each operator to decide. GSMA may provide further guidelines to the operators' community on the scopes recommended for 5GC deployments”.

It means that all the following work corresponding to this CVD is out of scope of 3GPP.

GSMA LS C4-213261: "Prevention of attacks on sliced core network". (https://www.3gpp.org/ftp/tsg_ct/WG4_protocollars_ex-CN4/TSGCT4_104e_meeting/Docs/C4-213261.zip).

SA3 Replied in S3-211527 (SA3 #103e) that “SA3 would like to acknowledge that SA3 specifications could be further improved to avoid misinterpretation of how access tokens are issued by the NRF and consumed by an NF producer”.

And then in the following meeting (SA3 #104, #105, #106, #107, #108, #109), the CR 1429 on TS 33.501 (S3-223860) in #109 meeting was approved from R18 to fix this problem in the end, the content of  which requires new operations in the NRF and NFp, and is not align with the conclusion of TR 29.857.

SA3 recommended CT4 to take CR S3-223860 into account to mitigate the problem identified by the GSMA. Therefore, the proposal on the enhancement of the existing OAuth token by CT4 in TR 29.857 is not necessary.

2	Actions

To CT4

ACTION: 	SA3 kindly asks CT4 to take above information into account.

3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)