3GPP TSG-SA3 Meeting #117	S3-243068

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Address EN for solution #1

# Document for:	Approval

Agenda Item:	5.15

1	Decision/action requested

Approve the pCR to TR 33.754[1].

2	References

[1]	3GPP TR 33.754: "Study on security aspects for Multi-Access (DualSteer + ATSSS Ph-4) ".

3	Rationale

This contribution addresses the EN for solution #1 and proposes few editorial changes.

4	Detailed proposal

*** Start of 1st Change ***

6.1 Solution #1: Using 3GPP security context to derive authentication pre-shared key for NIN3A

6.1.1	Introduction

This solution addresses key issue #1.

This solution gives the key architecture for ATSSS-Lite scenario and lists some options about how to derive and obtain the authentication key used for non-3GPP access between UE and UPF.

6.1.2	Solution details

In the simplified ATSSS architecture, there is an assumption that UE will establish connection with network and establish PDU session using 3GPP access. Considering UE and network already generated shared security context during the registration procedure via 3GPP access. A sub-level shared key can be generated and used as a pre-shared key for IKEv2 procedure.

The key hierarchy defined in TS 33.501[3] for this scenario can be extended as follows:

Figure 6.1.2.1 Key hierarchy generation for ATSSS-lite scenario

A new key KUPF is derived from KAMF as depicted in Figure 6.1.2.1. AMF derives the KUPF from KAMF during the PDU session establishment procedure over 3GPP access as showing in clause 6.1.2 of S3-241982.

When deriving a KUPF from KAMF, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xXX,

-	P0 = PDU session ID,

-	L0 = length of PDU session ID,

-	P1 = Uplink NAS COUNT,

-	L0 = length of Uplink NAS COUNT.

The input key KEY shall be KAMF.

Regarding with when SMF gets the KUPF in the case the SMF needs to transfer the KUPF, there are  several options:

- Option 1:  AMF sends KUPF to SMF in the CreateSMContext Request message, SMF selects the UPF and sends the key KUPF to UPF.

- Option 2: SMF requests the key from AMF and based on the reply, the SMF further sends it to UPF during the PDU session establishment procedure.

Regarding with when UPF gets the KUPF, there are several options as well:

- Option 1: UPF requests the key from SMF based on the request from the UE via NIN3A.

- Option 2: SMF sends the key to the UPF during the PDU session establishment procedure.

- Option 3: SMF requests the key from AMF and AMF sends the key to UPF during the PDU session establishment procedure.

6.1.3	Evaluation

TBD

There is no direct interface between UPF and AMF, AMF directly sends KUPF to UPF has 5G system impact.

*** End of 1st Change ***