3GPP TSG-SA3 Meeting #117	S3-243098

Maastricht, Netherlands  19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | DRAFTCR | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Propose solution in TNAP and TWIF case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Fast BSS solution was agreed as basis for TNAP reconnection optimization, thus this contribution propose to add this procedure . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new content for TNAP reconnection optimization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X.Y(new), X.Y.1(new), X.Y.2(new), X.Y.3(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF 1st CHANGES ****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[113]	RFC 9110: "HTTP Semantics".

[114]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[x]	IEEE Std 802.11™-2020 Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

**** End OF 1st CHANGES ****

**** START OF 2nd CHANGES ****

X.Y	Optimization for Non-3GPP reconnection

X.Y.1 General

Comparing to the procedure specified in clause 7A.2 in the present document, the procedure specified in this clause is an optimization allowing the UE to reconnect to a new non-3GPP access without involving core network. This optimization is based on the Fast BSS Transition protocol as described in 802.11[x].

X.Y.2 Authentication procedure for reconnecting to a Non-3GPP access

Figure X.Y.2-1 Fast BSS based Authentication Procedure for trusted non-3GPP access reconnection

1. It is assumed that the UE and the network has already performed the procedures including authentication as specified in clause 7A.2 and hence both UE and TNGF has obtained a K_TNGF key。

2. The UE and TNGF shall generate the KFT as described in clause A.X. KFT is used as the Master PMK (MPMK) for Fast BSS based protocol as described in 802.11[x]. Due to the Fast BSS protocol has different variants, the KTF may further generate PMK-R0 either in the step1 or in step4.

3. The UE reconnects to target AP Fast BSS Transition protocol as described in 802.11[x].

4. The UE and the target AP may establish the WLAN security by 4-way handshake.

X.Y.3 key hierarchy

The key hierarchy described in clause 6.2.1 applies, with the following changes:

The MPMK derived for non-3GPP access reconnection using BSS transition procedure as specified in 802.11[x] is called KFT in the context of trusted access.

PMK-R0 and PMK-R1 derivation shall comply with  BSS transition procedure as specified in 802.11[x].

The keys KFT is derived as described in clause A.X.

Figure X.Y.3-1: Link between the 5G and FT key hierarchies

**** End OF 2nd CHANGES ****

**** START OF 3rd CHANGES ****

A.X	KFT derivation function

When deriving a KFT from KTNGF the following parameters shall be used to form the input S to the KDF.

-	FC = 0x??

-	P0 = 0x03

-	L0 = length of 0x03

The input key KEY shall be KTNGF.

****** END OF CHANGES ****