3GPP TSG-SA3 Meeting #117	S3-243112

Maastricht, Netherlands  19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | DraftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Expired Certificate checking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei; HiSilicon, CAICT, CTCC, Nokia, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2024-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of a new testcase related to the threat of certificate expiry checking intorduced in a parallel contribution S3-243114 to the gNB network product class in TR 33.926. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new testcase for certificate expiry checking by gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete SCAS coverage of the certificate enrolment functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* 1st of Change*************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	Void

[5]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[7]	3GPP TS 23.501: "System Architecture for 5G System (5GS)".

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description".

[9]	3GPP TS 33.523: "5G Security Assurance Specification (SCAS); split gNB product classes".

[x]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[Y]	IETF RFC 4210: "Internet X.509 Public Key Infrastructure Certificate Management Protocol".

************* End of 1st Change*************

************* 2nd of Change*************

4.2.2.1.x	Expired Certificate checking

Requirement Name: Expired Certificate checking at base station

Requirement Reference: TS 33.310 [x], clause 7.2

Requirement Description:

Certificate Management Protocol v2 (CMPv2) [Y] may be the supported protocol to provide certificate lifecycle management capabilities for TLS entities. All TLS entities and TLS CAs may support initial enrolment by the TLS entity to the TLS CA via CMPv2, i.e. receiving a certificate from the TLS CA, and updating the key of the certificate via CMPv2 before the certificate expires.

Threat References: TBD

Test case:

Test Name: TC_EXPIR_CERT_CHCK

Purpose:

Verify that the gNB can check whether the certificates issued by operator CA are about to expire and to act accordingly.

Pre-Conditions:`

-	If the gNB under test does not support handling certificates as defined in clause 9 of TS 33.310[x], this test does not apply.

-	The gNB network product is connected in emulated/real network environments.

-	A TLS CA may be emulated, if needed.

-	The gNB is configured the necessary information to connect with the CA server.

-	Optionaly, the vendor may provide necessary informatin in a document, e.g. describing how to handle the case when a gNB checks the operator certificate is about to be expired.

Execution Steps

1. The tester configures the gNB with certificates that is assigned by operator CA.

2. The tester configures the UTC timer of gNB to the time when the certificate is about to be expired.

3. The gNB initiates the CMPv2 procedure to get the new operator certificate.

Expected Results:

-	The gNB raises an alarm or requests a new certificate from the CA.

-	The gNB received a new operator certificate.

Expected format of evidence:

The logs and the communication flow in a .pcap file.

************* End of Change*************