3GPP TSG-SA3 Meeting #117	S3-243129

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, Hisilicon

# Title:	New Solution on user authentication with user certificate of KI1

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

It is proposed to approve these changes for addressing key issue#1 in TR 33.700-32.

# 2	References

# 3	Rationale

A new solution addressing the KI#1 is proposed based on the EAP framework and a user identity certificate generated by an authorized UE for user authentication.

# 4	Detailed proposals

*** Start of Changes ***

## 6.Y	Solution #Y: User Authentication with Certificate Generated by an authorized UE

### 6.Y.1	Solution Introduction

This solution addresses KI#1: Authentication and Authorization of Human User ID. This solution is based on the EAP framework similar to NSSAA as in TS 33.501 [3]. Specifically, this solution enables linking a UE’s subscription to the user, through creating a user identity certificate by the UE and using the user identity certificate in EAP framework (e.g., EAP-TLS) for user authentication.

In the solution, the following logical function is introduced:

Identity Certificate Authentication Function (ICAF): assuming a new logical function responsible for verifying a public key certificate generated by a UE for a user. It can be collocated with the existing NF, e.g., AUSF.

### 6.Y.2	Solution Details

After UE primary authentication, the User ID is sent in a User Registration message to the AMF. The AMF triggers the user authentication procedure with the EAP framework (e.g., EAP-TLS) where a user identity certificate generated by UE is used as the client certificate.

Figure 6.Y.2-1: User authentication procedure with user identity certificate in EAP framework (e.g., EAP-TLS)

- The UE is provisioned with a key pair, i.e., a private key and the corresponding public key from the 3GPP network, used to generate a user identity certificate. The key pair provisioning is out of scope of this document and left to operator’s implementation. The ICAF is provisioned with the public key.

- The UE registers to the 3GPP network. The primary authentication is performed as described in clause 6.1 in TS 33.501 [3].

- The UE locally generates a user identity certificate signed by the private key provisioned in Step 0. The user identity certificate includes the public key of the user, User ID and a valid period with the signature of the UE.

- The UE sends a User Registration Request message to the AMF with the user ID.

- The AMF triggers the user authentication procedure by sending a User Authentication Request including the User ID to ICAF.

NOTE: AMF acts as an EAP authenticator.

- Mutual authentication between the User and the network using the EAP-TLS. The first part is the User authenticating the network. A network certificate is provided and the UE is able to verify the network certificate. The second part is the network authenticating the User and the ICAF can verify the UE generated user identity certificate by using the public key of the UE.

NOTE: ICAF acts as an EAP server.

- The ICAF sends a user authentication response to the AMF including the User ID and the authentication result.

- The AMF sends the authentication result to the UE.

### 6.Y.3	Evaluation

This solution addresses KI#1 based on the EAP framework as NSSAA in TS 33.501 [3]. Specifically, the EAP-TLS is used to authenticate the user with a user identity certificate. The user identity certificate is generated by an authorized UE to bind the user with the UE’s subscription. The binding can be done offline even if the UE is out of service.

The impacts are:

- UE: it needs the capability of creating a public key certificate.

- Core Network: a new logical functionality (e.g., ICAF) is added in order to provide user authentication.

*** End of Changes ***