3GPP TSG-SA3 Meeting #117	S3-243265

Maastricht, Netherlands  19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0076 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completing the Privacy Check of n UEs for Service Exposure to Client UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, OPPO, Phillips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The authorization procedure for service exposure to the Client UE sepcified in clause 6.3.6.3 of TS 33.533 covers two authorization phases. The first phase is to authorize the Client UE for service access, which is performed during PC5 link establishment. The next phase is to check UE privacy after PC5 link establishment with successful service access authorization. These two phases need to be elaborated in a clearer manner to avoid misinterpretation in other WGs (e.g. SA2). Hence, it is proposed to update the current description in 6.3.6.3 accordingly. Addtionally, in current version of TS 33.533, the handling of privacy check of n UEs is specified for network-based operation procedure in clause 6.3.5.0. However, the handling of privacy check of n UEs for UE-only operation in 6.3.6.3 is not completed yet. Hence, it is proposed to add more text on handling of privack check of n UE for UE-only operation to complete the procedure. In UE-only procedure specified in clause 6.8 of TS 23.586, a SL Positioning Server UE can request the location information of other UEs in steps 9 and 10, via issuing an RSPP supplementary services request via UE1 (i.e. Target UE) which selected the Server UE. This could also impose privacy risk, since the Server UE will have all the required information from other UEs for location calculation. Hence, it is proposed to take into account of the SL Positoning Server UE if it is selected before UE privacy check for UE-only operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the current description in 6.3.6.3 to elaborate the different phases of authorization for service exposure to the Client UE. Added additional text to complete the procedure for privacy check of n UEs for service exposure to the Client UE via PC5. Added details to clause 6.3.7 on how a local privacy check is performed in case a SL Positioning Server UE is selected for UE-only operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description on the authorization procedure for service exposure to the Client UE in clause 6.3.6.3 has led to some misunderstanding in other WGs. The privacy check of n UEs for UE-only operation is unspecified, leading to imcomplete specification. Privacy check of exposing location information to a SL Positioning Server UE in UE-only operation is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6.3, 6.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

6.3.6.3	Authorization procedure for Ranging/SL positioning service exposure through PC5

For Ranging/SL Positioning service exposure through PC5 (i.e. clause 6.7.1.1 of TS 23.586 [2]), the SL Positioning Client UE authorization is triggered by the Reference/Target UE during PC5 link establishment. The authorization for service access can be performed by the network via the SLPKMF for ProSe capable UEs according to clause 6.4.3.3 or by the Reference/Target UE if the authorization information is available in the UE.

If the Client UE is not authorized to access Ranging/SL Positioning service, the PC5 link between the Client UE and the Reference/Target UE shall not be established.

For UE-only operation or before triggering SL-MO-LR for Network based operation, to preserve the privacy of the Target/Reference UEs, the UE1 (i.e. Target UE or SL Reference UE receiving the Ranging/SL positioning request as in clause 6.8 of TS 23.586 [2]) shall send a supplementary RSPP signalling message to UE2/../UEn respectively to trigger privacy check for Ranging/SL positioning service exposure through PC5. The supplementary RSPP signalling message shall include Client UE's user info ID that is received by UE1 from the Client UE. The UE1 and UE2/../UEn shall perform UE privacy check as described in clause 6.3.7 to determine whether their location related information can be exposed to Client UE.

If none of the n UEs grants permission for exposure to the Client UE, the UE1 shall reject the Ranging/SL Positioning service request from Client UE. If all of the n UEs grant permission for exposure to the Client UE, the UE1 shall proceed with the Ranging/SL positioning service request from the Client UE. If part of the n UEs return positive privacy check results and part of the n UEs return negative privacy check results, UE1 shall determine to accept or reject the service request based on the privacy check results of the n UEs and a criterion up to UE implementation (e.g. a rule from application layer).

If the Client UE is authorized as per UE privacy check, UE1 shall send the Ranging/SL positioning request to the selected LMF or SL Positioning Server UE by including only the identities of UE2/../UEn returning positive privacy check results (step #6 of TS 23.586 [2] clauses 6.7.1.1 and 6.8). If the Client UE is not authorized as per UE privacy check, the Ranging/SL Positioning service request shall be rejected by UE1.

*************** Start of the 2nd Change ****************

6.3.7	Procedure of UE privacy verification for UE-only operation

For UE-only Operation in which the network is not involved in Ranging/Sidelink positioning, the authorization for UE privacy is based on the local configured privacy verification information to determine whether its location related information can be exposed to the UE(s) (e.g. peer UE or SL Positioning Server UE) indicated in the supplementary RSPP signalling message. To enable privacy check of exposure to a SL Positioning Client UE via the peer UE, the peer UE shall also include the user info ID of the SL Positioning Client UE in the supplementary RSPP signalling message to the UE from which the location information or related results are to be exposed.

UE privacy check against SL Positioning Server UE only applies to UE-only operation, and is performed only when the Server UE is selected by the Target UE (as per clause 6.8 step 5 in TS 23.586 [2]) before sending the supplementary RSPP signalling message for privacy check. The SL Positioning Server UE may be reselected based on the privacy check results of the Target/Reference UEs.

If the privacy verification information allows location exposure, the UE (e.g. Located UE) accepts the request to expose its location related information and proceeds. If the UE has no local privacy verification information or the privacy verification information disallows location exposure, the UE shall return privacy check reject message.

To enable privacy check of exposure to a SL Positioning Client UE via the peer UE, the peer UE shall include the user info ID of the SL Positioning Client UE in the supplementary RSPP signalling message to the UE from which the location information or related results are to be exposed.

*************** End of the Changes ****************