3GPP TSG-SA3 Meeting #117 	S3-243285

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Indian Institute of Technology Bombay

# Title:	Discussion on SS7/MAP interface at SMSF to define MAP filtering and protection requirements for Security Assurance Specifications for SMSF

# Document for:	Discussion

Agenda Item:	4.4

# 1	Decision/action requested

This discussion paper provides justifications for new test cases related to MAP/SS7 interface for SMSF SCAS.

# 2	References

[1]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS)".

[3]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[6] 				GSMA PRD FS.40: "5G Security Guide ".

[7]	3GPP TS 33.204: "Network Domain Security (NDS); Transaction Capabilities Application Part (TCAP) user security".

[8]	3GPP TS 33.210: "Network Domain Security (NDS): IP network layer security".

[9]	GSMA PRD FS.11: "SS7 Interconnect Security Monitoring and Firewall Guidelines".

# 3	Abbreviations

5GS	5G System

AMF	Access and Mobility Management Function

DEA	Diameter Edge Agent

DoS	Denial of Service

GTP	GPRS Tunneling Protocol

IP-SM-GW	IP Short Message Gateway

MAP	Mobile Application Part

MO	Mobile Originated

MT	Mobile Terminated

NDS	Network Domain Security

NSA	Non-Stand Alone

SBI	Service Based Interface

SEPP	Security Edge Protection Proxy

SGd	Service Gateway Diameter

SM	Short Message

SMS	Short Message Service

SMS-GMSC	Gateway MSC For Short Message Service

SMSF	Short Message Service Function

SS7	Signalling System No. 7

STP	Signalling Transfer Point

TCAP	Transaction Capabilities Application Part

TCAPsec	TCAP user security

TS	Technical Specification

UDM	Unified Data Management

# 4	Rationale

This paper provides insights into the external threats to the SS7/MAP interface of SMSF resulting in new test cases of filtering and protection.

# 5	Detailed Discussion

5.1 SS7/MAP interface of SMSF

Figure 1 provides an overview of the SMSF interfaces with other network entities in a 5G System (5GS). The figure has been developed using information from Section 4.1, Section 4.2, and Annex K of 3GPP TS 23.040 [1] and Section 4.4.2 of 3GPP TS 23.501 [2]. 3GPP TS 23.040 [1] mentions in Sections 4.1, 4.2 and Annex K that the interface between SMSF and SMS-GMSC/IWMSC and SMS Router is based on Mobile Application Part (MAP) protocol as specified in 3GPP TS 29.002 [3]. This figure has recently been added in 3GPP TS 33.926 [4].

Therefore, based on 3GPP TS 23.040 [1], it can be concluded that the realization of SMS in 5GS may necessitate the use of the MAP protocol when interconnecting with legacy networks like 4G and 3G. SS7/MAP interface has been highlighted in Figure 1.

Figure 1: Interface connections between SMSF and other network entities

5.2 On the requirements at SS7/MAP interface of SMSF

The following concerns related to the MAP interface specified in GSMA FS.40 [6] are relevant for SMSF interactions with other entities:

- Quoting from GSMA FS.40 [6]: “If SMS interworking is supported over Diameter End-to-End Security (DESS), then such inter-operator SMS shall be protected with integrity and confidentiality protection. However, if SMS interworking is supported over MAP, which is part of the SS7 protocol stack, no such integrity or confidentiality protection can be offered to protect the privacy of the 5G subscriber.” This indicates that there is no integrity and confidentiality protection in MAP. Therefore, we propose a test case to protect integrity and confidentiality by assuring implementation of TCAPsec [7] and/or NDS/IP [8] at SS7/MAP interface of SMSF.

- According to GSMA FS.40 [6], “SS7 provides no protection capabilities and use of screening functions in Signalling Transfer Points (STPs) and SS7 firewalls are needed to secure the SS7 signalling traffic between roaming partners.” Additionally, the document states, “For 5G deployments with the NSA-based architecture, roaming traffic is handled between 4G Core Networks with the result that the security of the roaming traffic is via SS7 and/or Diameter and, therefore, needs additional protection by screening functions and firewalls.”

- 3GPP TS 33.204 [7], clause 4.1 mentions “TCAPsec does not validate the TCAP user payload content. Message screening functions for particular message types may be needed on top of TCAPsec.”

Based on the points mentioned above, we propose filtering of messages at SS7 interface of SMSF.

5.3 What are the different categories of filtering required for MAP/SS7 interface at SMSF?

Clause 12 in 3GPP TS 29.002 [3] defines the short message service management services at the MAP application. The services which are applicable to SMSF in 5G system are enlisted in the below table.

| Service Name | Abbreviation | Opcode | Clause [3] |
| --- | --- | --- | --- |
| MAP-MO-FORWARD-SHORT-MESSAGE service | MO-FSM | 46 | 12.2 |
| MAP-ALERT-SERVICE-CENTRE service | ASC | 64 | 12.5 |
| MAP-MT-FORWARD-SHORT-MESSAGE service | MT-FSM | 44 | 12.9 |

GSMA FS.11 [9] recommends 3 different categories of rules for the SS7 Firewall in Annex B. For the MAP interface of SMSF, the below are the examples of filtering rules of the categories 1 and 2. For details on all filtering rules, please refer to Annex B of GSMA FS.11 [9].

| Title | Rule | Rationale |
| --- | --- | --- |
| Category 1: Interface unauthorized filter | for each inbound message, if MAP OpCode not in {44,46,64} THEN BLOCK | Block all messages which are not meant for SMSF |
| Category 2: Consistency filter | for each inbound message, if Calling Party Address at the SCCP layer!= the address at the MAP layer THEN BLOCK | Block all messages which don’t satisfy the consistency checks between the layers or in the messages themselves |

Below are a few examples illustrating how the lack of filtering support in above scenarios can result in a Denial of Service (DoS) attack on the SMSF. Figure 3 below illustrates how repeated incorrect address in SMS-GMSC address or spoofed SMS-GMSC in the MAP message can result in a DoS attack on the SMSF. Figure 4 below demonstrates how MAP messages which are not valid for SMSF can lead to a DoS attack on the SMSF.

STP

STP

SMSF

SMSF

Attacker as 
SMS-GMSC

Attacker as 
SMS-GMSC

Map-MT-Forward-Short-Message @invalid SMS-GMSC address

Map-MT-Forward-Short-Message @spoofed SMS-GMSC address

DoS

Map-MT-Forward-Short-Message @spoofed SMS-GMSC address

Map-MT-Forward-Short-Message @valid SMS-GMSC address

Map-MT-Forward-Short-Message @invalid SMS-GMSC address

Map-MT-Forward-Short-Message @spoofed SMS-GMSC address

DoS

Map-MT-Forward-Short-Message @spoofed SMS-GMSC address

Map-MT-Forward-Short-Message @valid SMS-GMSC address

Figure 3: Illustration for DoS attack in absence of category 2 filtering

Map application message @invalid message (Opcode = 34)

DoS

Map application message @invalid message (Opcode = 45)

Map application message @invalid message (Opcode = 65)

Map application message @invalid message (Opcode = 34)

DoS

Map application message @invalid message (Opcode = 45)

Map application message @invalid message (Opcode = 65)

SMSF

SMSF

STP

STP

Attacker as 
SMS-GMSC

Attacker as 
SMS-GMSC

Map application message @valid message (Opcode = 44)

Map application message @valid message (Opcode = 44)

Figure 4: Illustration for DoS attack in the absence of category 1 filtering

Therefore, based on our findings and the preceding discussion, it is evident that implementing effective filtering mechanisms at the MAP-based SS7 interface of the SMSF is crucial. Additionally, incorporating TCAPsec and/or NDS/IPsec will ensure that only authenticated and integrity-checked messages reach the SMSF. These measures will protect against potential DoS attacks and maintain the integrity and availability of the network at the application layer.

5.4 Proposal

IIT Bombay proposes to regard the filtering and protection of the MAP-based SS7 interface as a necessity to safeguard the availability and integrity of the MAP interface at SMSF, along with their corresponding test cases within the existing SMSF SCAS (TS 33.529 V0.5.0).